DUAL-MOTOR PMSM DYNAMOMETER PLATFORM FUNCTIONAL REQUIREMENTS & VALIDATION TI C2000 F28069M dual-inverter control, interfaces, verification, and acceptance

| Prepared by | Herder Elektronische Systemen |
| --- | --- |
| Document scope | System behavior, operating modes, external interfaces, safeguards, verification, and acceptance |
| Evidence basis | Editable Simulink models, custom ADC source, CAN interfaces, scripts, schematics, and retained verification evidence |
| Distribution | Customer-facing; selected source models and engineering evidence included |

DOCUMENT PURPOSE Defines the controlled functional, performance, interface, safety, verification, and acceptance requirements for the dual-motor PMSM dynamometer platform. Detailed implementation and efficiency characterization are controlled in the companion engineering report.

WHY THIS IS AN FRD This document defines what the released platform shall do, the interfaces it shall preserve, the evidence required to verify compliance, and the acceptance conditions for release. It is not an implementation walkthrough or a results report.

# Functional Requirements Document Framework

## Purpose, Scope, and System Boundary

This FRD governs the released dual-motor dynamometer platform at the level of externally observable behavior, interfaces, safeguards, evidence obligations, and acceptance. The companion Embedded Control & Characterization Report explains how the Simulink, CAN, ADC, logging, and analysis implementations satisfy this controlled requirement set.

## Users and Operational Scenarios

| User / scenario | Decision supported |
| --- | --- |
| Controls engineer | Modify target or host models without violating controlled interfaces or operating behavior. |
| Test engineer | Select verification methods, evidence, and acceptance criteria for a release. |
| Customer or reviewer | Understand system capability, boundaries, and demonstrated compliance. |
| Maintainer | Determine whether a proposed change requires FRD revision and re-verification. |

## Requirement Conventions

Mandatory requirements use “shall.” Each requirement has a unique identifier, verification method, acceptance basis, and release status. Descriptive text explains intent but does not replace the requirement statement.

| Status | Meaning |
| --- | --- |
| Verified | Objective evidence has been inspected and accepted. |
| Demonstrated | Behavior was observed on the retained bench evidence set. |
| Open | Evidence or qualification remains incomplete. |
| Not claimed | Capability is intentionally outside the public release boundary. |

# 1. System Role and Delivered Capability

The platform couples two encoder-equipped permanent-magnet synchronous machines on a common shaft. Motor 1 acts as the drive machine and establishes shaft speed. Motor 2 applies opposing or assisting q-axis current to create a controllable load. A TI C2000 F28069M target executes the real-time motor-control functions; host models provide supervisory commands, acquisition, and test orchestration.

Embedded target retains ownership of high-rate current and speed control.

Supervisory host supplies speed, load, enable, selection, and diagnostic commands.

Independent encoder channels preserve separate feedback for both machines.

CAN and host acquisition paths support repeatable command and evidence workflows.

Custom ADC source extends analog acquisition while generated build output remains excluded.

## 1.1 Normal Operating Sequence

A normal run begins with wiring and polarity inspection, low-risk deployment, phase-order and encoder-direction confirmation, low-speed enable, controlled speed/load application, data capture, and post-run evidence review. The host coordinates the sequence but does not become the real-time control clock.

## 1.2 Operating Modes

| Mode | Required behavior |
| --- | --- |
| Disabled / safe | Commanded torque production is inhibited and the system remains ready for inspection or configuration. |
| Drive-only | Motor 1 regulates shaft speed while Motor 2 remains at zero or bounded load command. |
| Loaded dynamometer | Motor 1 regulates speed while Motor 2 applies bounded opposing or assisting q-axis current. |
| CAN-supervised | Command words are received over CAN and mapped into existing control references and diagnostics. |
| Acquisition / review | Signals are captured and retained for verification and characterization without altering embedded timing. |

# 2. Functional Requirements

The following requirements define required behavior independent of the internal block arrangement. A compliant implementation may evolve, but these externally visible functions shall remain intact unless this FRD is revised.

| ID | Requirement | Verification | Status |
| --- | --- | --- | --- |
| SYS-001 | The embedded target shall execute the fast motor-control loops independently of host display or logging timing. | Inspection + functional test | Demonstrated |
| SYS-002 | The platform shall command one machine as the drive unit and the second as a controllable load or assist unit. | Functional test | Demonstrated |
| SYS-003 | The platform shall accept a supervisory speed reference for the drive machine. | Functional test | Demonstrated |
| SYS-004 | The platform shall accept a q-axis current or equivalent load reference for the load machine. | Functional test | Demonstrated |
| SYS-005 | The platform shall provide explicit enable and disable control for commanded operation. | Inspection + test | Demonstrated |
| SYS-006 | The platform shall maintain independent encoder feedback paths for both machines. | Inspection + feedback test | Demonstrated |
| SYS-007 | The released workflow shall support repeatable parameter setup, capture, and post-run review. | Procedure review | Verified |

## 2.1 Requirement Rationale

SYS-001 prevents nondeterministic host refresh or file I/O from becoming part of the current-control timing chain. SYS-002 through SYS-006 preserve the dual-machine dynamometer role and the feedback separation required to diagnose either machine. SYS-007 establishes repeatability as a release obligation rather than an informal operator practice.

# 3. External Interface Requirements

Interface requirements control the boundaries most likely to cause hardware damage, command ambiguity, or invalid evidence: DC input, motor phases, encoders, CAN, deployment connection, and analog acquisition.

| ID | Requirement | Verification | Status |
| --- | --- | --- | --- |
| IF-001 | The inverter stack shall be supplied from 24 V DC with verified polarity and current limiting before enable. | DMM check + startup test | Demonstrated |
| IF-002 | Each motor shall remain connected consistently to its assigned three-phase inverter output. | Continuity / wiring inspection | Demonstrated |
| IF-003 | Motor 1 and Motor 2 encoder channels shall remain electrically and logically distinct. | Pin inspection + feedback test | Demonstrated |
| IF-004 | The CAN bus shall provide CANH, CANL, reference ground, and appropriate termination. | Resistance + communication test | Demonstrated |
| IF-005 | The target shall retain the debug/deployment interface required for configuration and diagnostics. | Connection inspection | Demonstrated |
| IF-006 | The custom ADC integration shall configure the released simultaneous-sampling channels, trigger sources, acquisition windows, and end-of-conversion interrupt routing. | Source inspection | Verified |

## 3.1 Controlled CAN Contract

| Item | Public requirement |
| --- | --- |
| Command identifier | Standard CAN identifier 0x100. |
| Command payload | Eight-byte payload containing four 16-bit supervisory command words. |
| Return identifier | Standard CAN identifier 0x101 for command echo or diagnostic return. |
| Control integration | Decoded command words feed the existing reference, enable, selection, and FOC signal paths. |

## 3.2 Controlled ADC Contract

| SOC pair | Inputs | Trigger | Completion |
| --- | --- | --- | --- |
| SOC12 / SOC13 | ADCINA2 / ADCINB2 | ePWM1 | EOC13 -> ADCINT5 |
| SOC14 / SOC15 | ADCINA1 / ADCINB1 | ePWM1 | EOC15 -> ADCINT9 |

The custom source establishes the additional analog acquisition path. Detailed register-level implementation and source excerpts are maintained in the companion engineering report and embedded-control folder.

# 4. Evidence and Data Requirements

The public release must preserve enough evidence to confirm required behavior without distributing private raw datasets, generated firmware, or vendor manufacturing packages.

| ID | Requirement | Verification | Status |
| --- | --- | --- | --- |
| DAT-001 | The release shall expose the speed, current, voltage, torque-related, power-related, and diagnostic quantities required by the approved verification workflow. | Model/script inspection | Demonstrated |
| DAT-002 | Published evidence shall remain traceable to retained source models, scripts, or approved outputs. | Repository inspection | Verified |
| DAT-003 | Measured evidence shall distinguish observed operating regions from unmeasured or unsupported regions. | Figure/report inspection | Verified |
| DAT-004 | Interrupted or incomplete acquisition shall not be represented as complete evidence. | Procedure inspection | Verified |
| DAT-005 | The public release shall exclude generated code, binaries, private raw data, and institution-specific instructional material. | Release inspection | Verified |

Efficiency methodology and maps are not part of this FRD. They are contained only in the Embedded Control & Characterization Report. This document controls the evidence obligations and acceptance basis, not the interpretation of characterization results.

# 5. Safety, Constraints, and Release Limitations

| ID | Requirement | Verification | Status |
| --- | --- | --- | --- |
| SAFE-001 | DC polarity shall be verified before inverter power is applied. | Inspection | Required |
| SAFE-002 | Initial operation after wiring or software change shall begin at low speed and bounded load. | Procedure review | Required |
| SAFE-003 | Software disable shall not be represented as an independent safety function. | Document review | Verified |
| SAFE-004 | Rotating components shall be secured and kept clear during operation. | Procedure review | Required |
| REL-001 | The release shall not claim calibrated metrology, production safety certification, EMC qualification, or long-duration thermal qualification. | Document review | Verified |

## 5.1 Assumptions and Dependencies

Compatible MATLAB/Simulink and C2000 support packages are available.

Vendor controller and inverter documentation is used to verify board-revision pin assignments.

Bench wiring, motor mounting, and coupler integrity are checked before operation.

Instrumentation calibration remains outside the public release unless separately documented.

# 6. Verification and Acceptance Plan

Verification confirms that each requirement is satisfied by inspection, analysis, demonstration, or test. Validation confirms that the integrated platform supports its intended drive/load, supervisory-control, and evidence-generation use cases.

| Method | Application |
| --- | --- |
| Inspection | Review source models, custom C source, wiring references, repository contents, and document statements. |
| Analysis | Evaluate equations, signal definitions, and consistency of the evidence workflow. |
| Demonstration | Observe integrated command, feedback, CAN, and acquisition behavior. |
| Test | Apply controlled speed/load commands and confirm required responses and interface operation. |

## 6.1 Requirements Traceability Matrix

| Requirement | Retained evidence | Status |
| --- | --- | --- |
| SYS-001 | Target model and deployment evidence | Demonstrated |
| SYS-002 | Dual-machine target/host models | Demonstrated |
| SYS-003 | Host reference path | Demonstrated |
| SYS-004 | Load-command path | Demonstrated |
| SYS-005 | Enable path | Demonstrated |
| SYS-006 | Encoder interface evidence | Demonstrated |
| SYS-007 | Setup and capture scripts | Verified |
| IF-001 | Power-up procedure | Demonstrated |
| IF-002 | Interface summary | Demonstrated |
| IF-003 | Connector map and feedback test | Demonstrated |
| IF-004 | CAN architecture and bus test | Demonstrated |
| IF-005 | Deployment workflow | Demonstrated |
| IF-006 | F28069M_ADC_Custom_Config.c | Verified |
| DAT-001 | Models and logging scripts | Demonstrated |
| DAT-002 | Repository evidence map | Verified |
| DAT-003 | Companion characterization report | Verified |
| DAT-004 | Acquisition procedure | Verified |
| DAT-005 | NOTICE.md and ZIP inspection | Verified |

## 6.2 Acceptance Basis

A release is acceptable when all mandatory requirements are verified or explicitly bounded as open, the repository passes release inspection, report references resolve to retained evidence, and no excluded private or institution-specific materials are present.

# 7. Document Control and Release Gate

| Release check | Acceptance criterion |
| --- | --- |
| Requirement quality | Identifiers are unique; mandatory statements are atomic and use “shall.” |
| Traceability | Every mandatory requirement maps to a verification method and retained evidence. |
| Document separation | Efficiency characterization appears only in the companion engineering report. |
| Public boundary | Generated code, binaries, private raw data, and instructional material are excluded. |
| Visual QA | The DOCX and PDF are rendered and reviewed page by page. |
| Archive QA | The ZIP opens at repository root and passes integrity testing. |

Revision 2.2 applies the actual controlled FRD template structure, cover, typography, headers, table system, and document-control hierarchy from the approved Dual-Core Optical Motion Testbed FRD package.