DUAL-MOTOR PMSM DYNAMOMETER PLATFORM EMBEDDED CONTROL & CHARACTERIZATION C2000/Simulink implementation, custom ADC integration, CAN, acquisition, and measured results ENGINEERING REQUIREMENTS DOCUMENT ¦ REVISION 2.2 ¦ 5 AUGUST 2026

| Prepared by | Herder Elektronische Systemen |
| --- | --- |
| Document scope | Embedded implementation, editable models, custom firmware, acquisition workflow, and characterization evidence |
| Evidence basis | Simulink source models, handwritten F28069M ADC source, MATLAB tooling, plots, and interface references |
| Distribution | Customer-facing; controlled source models, custom firmware, and engineering evidence |

DOCUMENT PURPOSE Documents how the platform was implemented and what the retained engineering evidence demonstrates. Functional requirements and verification status remain controlled by the companion FRD.

# 1. Engineering Overview

The implementation combines model-based embedded control with a small, clearly identified handwritten peripheral layer. Editable Simulink models define the real-time dual-motor application, CAN interfaces, current-control subsystem, and supervisory hosts. A custom C source file extends the F28069M ADC configuration for additional simultaneous measurements. MATLAB utilities configure tests, capture sessions, and process the retained efficiency evidence.

COMPANION RELATIONSHIP Requirement IDs and verification status are controlled by the FRD. This report supplies implementation detail and evidence interpretation only.

# 2. Repository Source Inventory

| Layer | Public path | Purpose |
| --- | --- | --- |
| Target application | models/simulink/target/C2000_Dual_Motor_CAN_Target.slx | Editable real-time application model |
| Host models | models/simulink/host/*.slx | Efficiency, drive-cycle, and Speedgoat/CAN supervision |
| Subsystem models | models/simulink/subsystems/*.slx | CAN receive, CAN transmit, and current control |
| Custom firmware | embedded-control/custom-firmware/F28069M_ADC_Custom_Config.c | Handwritten register-level ADC configuration |
| MATLAB tooling | host-software/setup, logging, analysis | Configuration, capture, export, plotting, and processing |
| Evidence | results/*.png | Raw signals, filters, measured bins, density, and contours |

# 3. Embedded Control Architecture

Figure 1. Control and telemetry architecture used by the public implementation.

The C2000 target owns the high-rate field-oriented control loops and machine feedback. The host supplies supervisory references and test sequencing but does not act as the current-loop or speed-loop scheduler. Motor 1 operates as the drive machine. Motor 2 applies opposing or assisting q-axis current to create a controllable load condition.

## 3.1 Simulink model roles

| Model | Engineering role |
| --- | --- |
| C2000_Dual_Motor_CAN_Target.slx | Integrated C2000 target model containing dual-machine control, feedback, CAN paths, and deployable target logic. |
| Current_Control.slx | Reusable current-control implementation supporting the target FOC path. |
| CAN_Receive.slx | Receives and unpacks the supervisory command payload. |
| CAN_Transmit.slx | Returns command echo or selected telemetry for diagnostic separation. |
| Efficiency_Mapping_Host.slx | Commands repeatable speed/load points and supports efficiency acquisition. |
| Drive_Cycle_Host.slx | Runs time-varying speed and load profiles. |
| Speedgoat_CAN_Host.slx | Provides the real-time supervisory CAN endpoint. |

# 4. Custom F28069M ADC Integration

The public release includes handwritten source rather than only generated support code. F28069M_ADC_Custom_Config.c configures two additional simultaneous ADC pairs for the measurement path and integrates them with the target application.

| SOC pair | Analog inputs | Trigger | Acquisition window | Interrupt routing |
| --- | --- | --- | --- | --- |
| SOC12 / SOC13 | ADCINA2 / ADCINB2 | ePWM1 | 7 ADC clocks | EOC13 → ADCINT5 |
| SOC14 / SOC15 | ADCINA1 / ADCINB1 | ePWM1 | 7 ADC clocks | EOC15 → ADCINT9 |

OWNERSHIP AND ACCURACY The custom file is presented as register-level peripheral integration. Generated C/C++ support files and TI library internals are excluded; the release does not claim a ground-up ADC driver.

# 5. CAN Command and Diagnostic Path

The supervisory transport uses a standard CAN command frame containing four 16-bit words. The receive subsystem decodes references and control state into the existing target signal paths. A separate return identifier carries command echo or selected diagnostic telemetry, helping distinguish physical-link faults from target-logic faults.

| Item | Public definition | Implementation role |
| --- | --- | --- |
| Command ID | 0x100 | Four 16-bit command words in an 8-byte standard frame |
| Return ID | 0x101 | Command echo or selected diagnostic telemetry |
| Target integration | CAN receive and unpack | Decoded signals feed reference, enable, selection, and FOC paths |
| Diagnostic purpose | Return traffic | Separates bus-level failure from model-level interpretation |

# 6. Host Software and Acquisition Workflow

efficiency_test_setup.m configures repeatable speed/load sweeps and opens the neutral customer-facing host model.

drive_cycle_setup.m configures time-varying command profiles without school or course references.

save_debug_session.m exports timeseries, constants, metadata, and a session README.

post_dyno_export.m and plot_all_dyno_runs.m organize retained datasets and produce reviewable plots.

motor_efficiency_processing.m performs signal alignment, power calculations, validity filtering, binning, density review, and contour generation.

# 7. Characterization Method

The processing chain intentionally exposes intermediate decisions. Raw command and feedback signals are aligned, forced or invalid values are identified, physics-based masks are applied, retained operating points are reviewed, measured bins are formed, sample counts are inspected, and interpolation is presented only after measured coverage is visible.

Figure 2. Raw speed, current, voltage, mechanical, and torque-related signals retained before filtering.

Figure 3. Explicit electrical-power, mechanical-power, torque, and combined validity masks.

# 8. Measured Evidence

Figure 4. Retained speed-torque operating points colored by calculated efficiency.

Figure 5. Measured-bin efficiency map; unpopulated regions remain visibly distinct.

Figure 6. Sample-count map used to judge operating-region density and confidence.

## 8.1 Controlled interpretation

Published speed sweeps approach 4000 RPM in the retained evidence.

Demonstrated torque-related operating points are concentrated roughly between 0.045 and 0.17 N·m.

Efficiency values are filtered using explicit power, torque, and bounded-efficiency criteria before binning.

Measured bins and sample density are the primary evidence used for public interpretation.

The results demonstrate bench integration and processing discipline, not calibrated product efficiency or regulatory qualification.

# 9. Physical Interface Evidence

Figure 7. BoosterPack-to-LaunchPad interface reference used during hardware integration.

The release also retains the applicable LaunchPad and BOOSTXL-DRV8305EVM public schematics and a concise interface summary. These references support pinout review and connection traceability without redistributing editable vendor manufacturing sources.

# 10. Limitations and Next Validation

| Area | Controlled statement |
| --- | --- |
| Torque and electrical calibration | Independent traceable calibration is outside the retained public evidence. |
| Thermal endurance | Long-duration thermal and derating qualification is not included. |
| Fault coverage | A complete independent protection chain and production fault tree are not claimed. |
| EMC and environment | Formal EMC, vibration, ingress, and environmental testing are not included. |
| Model dependencies | Opening and deploying the models requires compatible MATLAB/Simulink, C2000 support packages, and relevant licensed hardware blocks. |
| Physical demonstration media | A dedicated platform video or full-bench photograph should be added when approved and available. |