DUAL-MOTOR PMSM DYNAMOMETER PLATFORM FUNCTIONAL REQUIREMENTS & VALIDATION REPORT TI C2000 F28069M, dual-inverter control, interfaces, and verification Customer-facing technical report Revision 1.7 - 5 August 2026

| Prepared by | Herder Elektronische Systemen |
| --- | --- |
| Primary subject | System requirements, interfaces, verification status, and public release boundary |
| Evidence basis | Simulink source models, custom ADC source, CAN interfaces, scripts, figures, and schematics |
| Distribution | Customer-facing; selected source models, custom firmware, and engineering evidence included |

DOCUMENT PURPOSE This report defines what the public Dual-Motor PMSM Dynamometer Platform is required to do, how each requirement is verified, what evidence is retained, and which capabilities remain outside the release. Implementation details and measured-result interpretation are intentionally assigned to the companion engineering report.

# 1. Executive Summary

The platform is a coupled two-machine dynamometer built around a TI C2000 F28069M controller, two DRV8305 inverter stages, encoder feedback, supervisory host models, CAN communication, and a structured data-processing workflow. This FRD is deliberately concise: it defines mandatory behavior, interfaces, verification methods, and release limitations without duplicating the implementation narrative.

DOCUMENT BOUNDARY This FRD answers “what shall the system do and how is compliance shown?” The companion Embedded Control, CAN & Characterization Report answers “how was it implemented and what did testing show?”

# 2. Scope and System Boundary

Embedded field-oriented control for two coupled PMSMs.

Drive-machine speed command and load-machine q-axis current command.

Independent encoder feedback and target-derived acquisition signals.

Supervisory CAN command path and diagnostic return path.

Selected editable Simulink models, custom ADC C source, MATLAB utilities, public figures, and interface documentation.

## 2.1 Excluded claims and assets

No production functional-safety certification, calibrated torque traceability, EMC qualification, or environmental qualification.

No generated C/C++ code, binaries, caches, private raw MAT datasets, or workstation-specific build artifacts.

No vendor manufacturing packages, editable third-party PCB projects, Gerbers, or licensed software bundles.

No course instructions, student-facing lab material, answer keys, or internal university records.

# 3. System Context

Figure 1. Public system boundary: supervisory host, C2000 real-time target, dual inverter stages, and coupled drive/load machines.

# 4. Functional Requirements

| ID | Area | Requirement | Verification | Status |
| --- | --- | --- | --- | --- |
| SYS-001 | Real-time control | The system shall execute fast motor-control loops on the embedded target independently of host display timing. | Inspection + functional test | Demonstrated |
| SYS-002 | Dual-machine operation | The system shall command one PMSM as the drive machine and the second as a controllable load or assist machine. | Functional test | Demonstrated |
| SYS-003 | Speed reference | The system shall accept a supervisory speed reference for the drive machine. | Functional test | Demonstrated |
| SYS-004 | Load reference | The system shall accept a q-axis current or equivalent load reference for the load machine. | Functional test | Demonstrated |
| SYS-005 | Enable control | The system shall provide explicit enable and disable control for commanded operation. | Inspection + test | Demonstrated |
| SYS-006 | Feedback | The system shall maintain independent encoder feedback paths for both machines. | Inspection + feedback test | Demonstrated |
| SYS-007 | Observability | The system shall expose speed, current, voltage, torque-related, power-related, and diagnostic quantities required by the public workflow. | Inspection + analysis | Demonstrated |
| SYS-008 | Repeatable workflow | The host workflow shall support repeatable setup, capture, export, and postprocessing. | Procedure review | Demonstrated |

# 5. Interface Requirements

| ID | Interface | Requirement | Verification | Status |
| --- | --- | --- | --- | --- |
| IF-001 | DC input | The inverter stack shall use a verified 24 VDC supply with current limiting during bring-up. | DMM + startup test | Demonstrated |
| IF-002 | Motor phases | Each PMSM shall be connected consistently to its assigned inverter phase outputs. | Continuity + spin test | Demonstrated |
| IF-003 | Encoder routing | Motor 1 and Motor 2 encoder channels shall remain electrically and logically distinct. | Pin inspection + feedback test | Demonstrated |
| IF-004 | CAN physical layer | The CAN bus shall provide CANH, CANL, reference ground, and appropriate termination. | Resistance + bus test | Demonstrated |
| IF-005 | CAN command | The target shall receive the four-word supervisory command payload on the documented command identifier. | Bus test | Demonstrated |
| IF-006 | CAN diagnostics | The target shall provide a return identifier for command echo or telemetry diagnostics. | Bus test | Demonstrated |
| IF-007 | Custom ADC acquisition | The target shall configure two additional simultaneous ADC input pairs using handwritten F28069M peripheral source. | Source inspection + acquisition test | Demonstrated |

Figure 2. BoosterPack-to-LaunchPad board-interface reference retained with the public release.

# 6. Data and Processing Requirements

| ID | Area | Requirement | Verification | Status |
| --- | --- | --- | --- | --- |
| DAT-001 | Mechanical power | The analysis shall compute mechanical power from torque and angular speed. | Equation inspection | Verified |
| DAT-002 | Electrical power | The dq-frame workflow shall compute electrical power from 1.5 × Vq × Iq for the retained implementation. | Equation inspection | Verified |
| DAT-003 | Validity filtering | The workflow shall apply explicit minimum-power, minimum-torque, and bounded-efficiency masks before binning. | Script + figure inspection | Demonstrated |
| DAT-004 | Measured bins | Published efficiency maps shall distinguish populated measured bins from unmeasured regions. | Figure inspection | Demonstrated |
| DAT-005 | Sample density | The release shall retain a sample-count view for operating-region confidence review. | Figure inspection | Demonstrated |
| DAT-006 | Traceability | Published figures shall be traceable to retained scripts or approved evidence outputs. | Repository inspection | Verified |

# 7. Safety and Release Constraints

Verify DC polarity before energizing either inverter stage.

Begin with low speed, zero load command, conservative limits, and accessible shutdown.

Confirm encoder direction and phase order before closed-loop sweeps.

Do not treat software disable as an independent safety function.

Do not represent the public platform as production-qualified or safety-certified.

# 8. Requirements Traceability Matrix

| Requirement | Verification method | Retained evidence | Status |
| --- | --- | --- | --- |
| SYS-001 | Inspection + functional test | C2000_Dual_Motor_CAN_Target.slx | Demonstrated |
| SYS-002 | Functional test | target and host models | Demonstrated |
| SYS-003 | Functional test | host setup models | Demonstrated |
| SYS-004 | Functional test | host setup models | Demonstrated |
| SYS-005 | Inspection + test | host/target models | Demonstrated |
| SYS-006 | Inspection + feedback test | board interface + target model | Demonstrated |
| SYS-007 | Inspection + analysis | target model + processing scripts | Demonstrated |
| SYS-008 | Procedure review | setup/logging/analysis scripts | Demonstrated |
| IF-001 | DMM + startup test | interface summary | Demonstrated |
| IF-002 | Continuity + spin test | interface summary | Demonstrated |
| IF-003 | Pin inspection + feedback test | board-interface figure | Demonstrated |
| IF-004 | Resistance + bus test | CAN model set | Demonstrated |
| IF-005 | Bus test | CAN_Receive.slx | Demonstrated |
| IF-006 | Bus test | CAN_Transmit.slx | Demonstrated |
| IF-007 | Source inspection + acquisition test | F28069M_ADC_Custom_Config.c | Demonstrated |
| DAT-001 | Equation inspection | motor_efficiency_processing.m | Verified |
| DAT-002 | Equation inspection | motor_efficiency_processing.m | Verified |
| DAT-003 | Script + figure inspection | 03_filter_masks.png | Demonstrated |
| DAT-004 | Figure inspection | 05_measured_efficiency_map.png | Demonstrated |
| DAT-005 | Figure inspection | 06_sample_count_map.png | Demonstrated |
| DAT-006 | Repository inspection | repository map and checksums | Verified |

# 9. Validation and Release Gate

| Validation objective | Acceptance basis | Status |
| --- | --- | --- |
| Coupled drive/load operation | Motor 1 establishes shaft speed while Motor 2 applies controlled loading. | Demonstrated |
| Efficiency workflow | Physics masks, measured bins, sample density, and contour outputs retained. | Demonstrated |
| CAN supervisory control | Command receipt and motor-spin validation retained in public models and documentation. | Demonstrated |
| Custom ADC path | Additional PWM-triggered simultaneous input pairs integrated with target application. | Demonstrated |
| Production qualification | Calibration, endurance, EMC, environmental, and independent safety validation. | Not claimed |

RELEASE GATE Publish only after PDFs are rendered page by page, report links resolve, the ZIP opens at repository root, and public files contain no school-specific lab instructions or private build artifacts.