EMBEDDED CONTROL, CAN & CHARACTERIZATION REPORT

TI Piccolo-Based Dual-Motor Dynamometer Testbed

PUBLIC ENGINEERING RELEASE  v1.4

| Document ID | HES-DYNO-SW-001 |
| --- | --- |
| Prepared by | Herder Elektronische Systemen |
| System basis | TI C2000 Piccolo F28069M dual-PMSM dynamometer testbed |
| Purpose | Describe the real-time control architecture, CAN migration, host workflows, data-conditioning method, measured results, and evidence limitations that support the companion FRD. |

DOCUMENT POSITIONING

Independent engineering documentation for a platform built with Texas Instruments development hardware. This document is not authored, approved, or sponsored by Texas Instruments. Product names and trademarks remain the property of their owners.

# 1. Document Relationship

The companion Functional Requirements & Validation Report is the authoritative public baseline for requirements, verification methods, status, and traceability. This report explains how the embedded, communications, host, and characterization evidence supports that baseline.

# 2. Engineering Outcome

An inherited dual-motor bench with mismatched connection documentation was converted into a repeatable embedded-controls and characterization platform. The work corrected physical phase and encoder interfaces, restored dual-machine FOC operation, stabilized host-target pacing, built fixed-dwell and compressed drive-cycle command generation, implemented a CAN command/response path, and produced reviewable efficiency-map evidence.

Figure 1. Control and data-flow architecture.

# 3. Real-Time Target Responsibilities

Execute the motor-control loop, PWM generation, project-specific ADC-synchronized acquisition, QEP position/speed capture, and command interpretation.

Maintain separate drive-machine and load-machine control paths.

Receive packed host commands over CAN and expose response/telemetry values.

Keep time-critical behavior independent of host plotting and GUI refresh timing.

# 4. CAN Host-Target Interface

| Direction | Identifier | Payload | Public role |
| --- | --- | --- | --- |
| Host to target | 0x100 | 4 x 16-bit values | Command transport: speed, load/current, enable/state, and selection/configuration fields. |
| Target to host | 0x101 | 4 x 16-bit values | Response/echo and selected target values for integration verification. |

The retained source evidence shows standard CAN mailbox configuration for these identifiers and byte packing/unpacking for four 16-bit values. Exact private scaling tables and the complete telemetry dictionary are not published.

# 5. Host Workflows

| Workflow | Purpose | Evidence |
| --- | --- | --- |
| Fixed-dwell efficiency test | Hold defined speed/load operating points long enough to accumulate valid samples. | efficiency_test_setup.m |
| Drive-cycle test | Create a bounded, compressed FTP-style speed command and corresponding load command. | drive_cycle_setup.m |
| Efficiency processing | Crop, align, screen, compute, bin, count, and visualize efficiency data. | motor_efficiency_processing.m |
| Run export and plotting | Save selected sessions and compare runs. | host-software utilities |

# 6. Characterization Method

Signals captured across selected debug configurations are cropped to a common interval and interpolated onto a common time base. The workflow computes electrical and mechanical power, applies validity masks, bins accepted samples by speed and torque, stores sample counts, and produces measured and interpolated views. Interpolated surfaces are explicitly presentation aids rather than additional measured evidence.

| Screening rule | Threshold |
| --- | --- |
| Electrical input power | > 10 W |
| Mechanical output power | > 3 W |
| Torque | > 0.045 N m |
| Efficiency | Finite and physically plausible |
| Speed/torque grid | 0-4000 rpm; 0-0.25 N m |

Figure. Raw signal review.

Figure. Cleaning overlay showing retained and rejected regions.

Figure. Validity masks before derived calculations.

Figure. Accepted measured operating points.

Figure. Measured-bin efficiency map.

Figure. Evidence density by bin.

Figure. Interpolated visualization.

Figure. Contour visualization of the interpolated surface.

# 7. Demonstrated Outcomes and Interpretation Boundaries

| Outcome | Supported statement |
| --- | --- |
| Physical recovery | Phase order and encoder/QEP mapping were corrected against the actual platform. |
| Control bring-up | Dual-machine closed-loop operation was restored on the F28069M target. |
| Communications | CAN command and response transport was demonstrated with retained mailbox and packing evidence. |
| Data quality | Non-finite and implausible samples are screened before map generation. |
| Traceability | Measured bins, sample counts, and interpolated views are retained as distinct evidence products. |
| Boundary | No production safety, calibrated metrology, EMC, environmental, or functional-safety qualification is claimed. |

# 8. Release Status

This report is released with FRD revision 1.4. The public repository is a capability showcase and engineering evidence package, not a turnkey firmware distribution. Selected sanitized Simulink examples are included; complete target models, binaries, private scaling tables, raw data, and generated implementation trees remain controlled.

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.1 | 2026-08-03 | HES | Reformatted in TI-inspired application-note style and aligned to the new FRD and traceability model. |
| 1.2 | 2026-08-03 | HES | Added selected sanitized Simulink examples and aligned the companion report to FRD revision 1.4; no substantive technical findings changed. |

# 9. Custom Low-Level ADC Integration

Most application behavior is represented by editable Simulink models, but the target also contains a focused custom C layer for F28069M ADC peripheral configuration. This source is included under embedded-control/custom-firmware/ and is not a renamed generated support file.

## 9.1 Added acquisition resources

| SOC pair | Inputs | Trigger | Completion | Role |
| --- | --- | --- | --- | --- |
| SOC0/SOC1 | ADCINA0 / ADCINB0 | ePWM1 | EOC1 -> ADCINT1 | Baseline synchronized pair |
| SOC2/SOC3 | ADCINA3 / ADCINB3 | ePWM4 | EOC3 -> ADCINT2 | Baseline synchronized pair |
| SOC4/SOC5 | ADCINA0 / ADCINB0 | Software | Polled / model-controlled | Software-triggered pair |
| SOC6/SOC7 | ADCINA3 / ADCINB3 | Software | Polled / model-controlled | Software-triggered pair |
| SOC12/SOC13 | ADCINA2 / ADCINB2 | ePWM1 | EOC13 -> ADCINT5 | Added instrumentation pair |
| SOC14/SOC15 | ADCINA1 / ADCINB1 | ePWM1 | EOC15 -> ADCINT9 | Added instrumentation pair |

The project-specific additions are SOC12/SOC13 and SOC14/SOC15. Both pairs are synchronized to ePWM1 so the additional instrumentation channels are sampled against target-side timing rather than host execution timing.

## 9.2 Integration boundary

The Simulink target remains the primary editable application/control source.

F28069M_ADC_Custom_Config.c provides the project-specific register-level SOC, trigger, acquisition-window, and ADC interrupt configuration.

MathWorks-generated ADC support files, TI device-support source, object files, and binaries remain excluded.

A compatible C2000 support package, device headers, and the referenced target model are required to compile and deploy the complete target.

## 9.3 Controlled engineering claim

The demonstrated contribution is custom C2000 ADC peripheral integration for two additional simultaneous analog-input pairs, including PWM-triggered sampling and end-of-conversion interrupt routing. The release does not claim a from-scratch ADC driver or ownership of TI/MathWorks support libraries.