Project Overview: Dual-Motor PMSM Dynamometer

# First Semester (Hardware Bring-Up)

Worked with the Simulink model: mcb_pmsm_foc_qep_dyno_f28069m (PMSM dual-motor dyno model)
Set up a dual-motor dynamometer system (drive motor + load motor)
 Fixed major hardware issues:
  - Incorrect three-phase motor wiring on original documentation
  - Incorrect encoder pin connections on original documentation
 Got the system to run correctly without hardware faults

# This Semester (Control, Timing, and Data)

Fixed real-time behavior:
  - Tuned SCI transmit and receive timing separately (see the Tools folder)
  - Enabled “simulation pacing” in Simulink to achieve smooth, real-time operation

Figure 1: SCI in block sample time

Tuned PI controllers:
  - Revealed proper Iq and Vq behavior for motor 1
  - Stabilized performance during sweeps

Figure 2: PI Controller

Built a repeatable test process (see software & Lab 1)
 Created a MATLAB script to combine and extensively clean data (see Lab 1)
 Generated Motor 1 efficiency maps from real test runs (see Lab 1)

# Current Status

System runs smoothly in real time, throttles with excess data
 Hardware is fully corrected
 Control signals for motor 1 (Iq, Vq) behave correctly
 Producing clean, usable efficiency results; now developing EPA drive cycle lab

Figure 3: FTP-75 pathway on host model

# Summary

First semester: fixed hardware so the dyno could run
This semester: fixed timing (SCI), tuned control loops, analyzed/cleaned data for real results to develop a motor efficiency map;

Summer: integrating EPA drivecycle onto platform; switching to CAN bus comms; enabling all debug signals over CAN; writing final software package