# Lab 1 Data Processing Documentation
Motor 1 Efficiency Mapping from Trial01-Trial04

Scope: This document explains exactly what happens to the Lab 1 data at each processing step and why each step exists. It is based on the MATLAB script data_processing04 / build_motor1_efficiency_trial01_04_FINAL.m.

# Pipeline Summary

| Stage | What happens | Why it is needed |
| --- | --- | --- |
| Raw load | Reads trial01-trial04 debug CSVs | Brings required channels into MATLAB. |
| Raw plot clipping | Clips only plotting copies | Lets the user inspect raw data without huge spikes hiding the normal signal. |
| Time crop | Keeps finite samples in the selected time window | Removes startup/shutdown regions if needed. |
| Common time alignment | Interpolates separate logs onto one time vector | Allows point-by-point efficiency calculation from separately recorded variables. |
| Vd zero | Sets Vd equal to zero vector | Matches the actual Simulink model for this Lab 1 test. |
| Basic sanity filter | Removes NaN/Inf and impossible values | Prevents corrupt samples from entering calculations. |
| Power and raw efficiency | Calculates Pelec, Pmech, and eta | Creates the physical quantities needed for the efficiency map. |
| Physics filter | Keeps only adequate power, torque, and efficiency range | Removes fake high efficiency from low-power or low-torque data. |
| Clean dataset | Applies final mask to all variables | Produces synchronized valid operating points. |
| Measured bin map | Bins RPM and torque and uses median efficiency | Creates honest measured efficiency map. |
| Interpolated map | Fills and smooths only near measured bins | Creates presentation-quality visualization without replacing measured data. |

# 1. Processing Goal

The purpose of Lab 1 data processing is to combine four separately collected debug-signal trials into a single usable Motor 1 efficiency dataset. Each trial contains a different pair of signals from the Simulink debug selector. The processing script loads the separate CSV files, aligns them in time, removes corrupt or non-physical samples, computes efficiency, and creates both measured and presentation-style efficiency maps.

The major issue is that the required efficiency variables are not all logged in one file. Speed, Iq, Vq, Pm, and Te are collected as separate debug selections, so they must be recombined carefully before efficiency can be calculated.

# 2. Required Trial Order from Debug Signal Selector

The data must be collected in the following order. The script assumes this exact mapping.

| Trial folder | Debug selector choice | debug_1.csv | debug_2.csv |
| --- | --- | --- | --- |
| trial01 | Mtr1: Speed ref & Speed feedback | Speed reference | Speed feedback |
| trial02 | Mtr1: Iq ref & Iq feedback | Iq reference | Iq feedback |
| trial03 | Mtr1: Vd & Vq | Vd signal (not used) | Vq signal |
| trial04 | Mtr1: Pm & Te | Mechanical power Pm | Torque Te |

speedTrial = "trial01";   % Mtr1 Speed ref & Speed feedback
iqTrial    = "trial02";   % Mtr1 Iq ref & Iq feedback
vdvqTrial  = "trial03";   % Mtr1 Vd & Vq, but Vd forced to zero
pmteTrial  = "trial04";   % Mtr1 Pm & Te

Why this matters: if the trial order is wrong, the script will combine the wrong physical variables. For example, if trial03 does not contain Vd/Vq, then debug_2.csv will not be Vq, and the efficiency calculation will be invalid.

# 3. Script Settings and What They Mean

startTimeSec = 0;
endTimeSec   = inf;

maxAbsRPM = 6000;
maxAbsIq  = 8;
maxAbsVq  = 200;
maxAbsTe  = 0.30;
maxAbsPm  = 1000;

minPelec  = 10;       % W
minPmech  = 3;        % W
minTorque = 0.045;    % N*m
etaMin    = 0.0;
etaMax    = 1.05;

rpmEdges        = 0:100:4000;
torqueEdges     = 0:0.01:0.25;
minCountPerBin  = 3;
useMedianInBins = true;

| Setting | Meaning | Reason used in Lab 1 |
| --- | --- | --- |
| startTimeSec, endTimeSec | Time range used from each CSV | Allows startup or shutdown regions to be excluded if needed. In the shown script, start is 0 and end is inf, so the full run is kept except invalid values. |
| maxAbsRPM | Maximum allowed absolute speed | Rejects corrupted RPM samples that are impossible for the test. |
| maxAbsIq | Maximum allowed absolute q-axis current | Rejects current spikes or logging artifacts. |
| maxAbsVq | Maximum allowed absolute q-axis voltage | Rejects voltage spikes or invalid data. |
| maxAbsTe | Maximum allowed torque | Rejects torque samples outside the expected range. Lab 1 torque only reaches about 0.2 N*m. |
| maxAbsPm | Maximum allowed mechanical power signal | Rejects impossible mechanical power spikes. |
| minPelec | Minimum electrical power threshold | Avoids dividing by tiny electrical power values, which can create spurious high efficiencies. |
| minPmech | Minimum mechanical power threshold | Rejects near-zero mechanical output regions where noise dominates. |
| minTorque | Minimum torque threshold | Removes the low-torque band that produced unrealistic high efficiency. |
| etaMin, etaMax | Allowed efficiency range | Rejects negative efficiency and values above realistic physical limits. |
| rpmEdges | Speed bin edges | Sets map columns at 100 RPM resolution from 0 to 4000 RPM. |
| torqueEdges | Torque bin edges | Sets map rows from 0 to 0.25 N*m at 0.01 N*m resolution. |
| minCountPerBin | Minimum samples per map cell | Removes bins that do not contain enough data to be trustworthy. |
| useMedianInBins | Median instead of mean | Median reduces the influence of remaining outliers in each bin. |

# 4. Step 1 - Load Raw Signals

The script loads the seven required raw signals from the four trial folders. Each call returns a time vector and one signal vector. No cleaning is applied at this stage.

[tRpmRefRaw, rpmRefRaw] = readTrialSignal(baseDir, speedTrial, 'debug_1');
[tRpmRaw,    rpmRaw]    = readTrialSignal(baseDir, speedTrial, 'debug_2');

[tIqRefRaw, iqRefRaw] = readTrialSignal(baseDir, iqTrial, 'debug_1');
[tIqRaw,    iqRaw]    = readTrialSignal(baseDir, iqTrial, 'debug_2');

% Ignore Vd channel. Use debug_2 only as Vq.
[tVqRaw, vqRaw] = readTrialSignal(baseDir, vdvqTrial, 'debug_2');

[tPmRaw, pmRaw] = readTrialSignal(baseDir, pmteTrial, 'debug_1');
[tTeRaw, teRaw] = readTrialSignal(baseDir, pmteTrial, 'debug_2');

Important: Vd from trial03/debug_1.csv is deliberately not loaded for processing. The actual Simulink model has Vd equal to zero for this Motor 1 efficiency test condition, so the script later creates Vd as an all-zero vector and uses Vq from debug_2.csv.

# 5. Step 2 - Plot True Raw Data with Visual Clipping Only

The first diagnostic plot is meant to show what the unprocessed data looks like. A few channels may contain large spikes that make the normal signal impossible to see, so the script makes temporary plotting copies and removes extreme values only from those plotting copies.

iqPlot = iqRaw;
vqPlot = vqRaw;
pmPlot = pmRaw;

iqPlot(abs(iqPlot) > 20)   = NaN;
vqPlot(abs(vqPlot) > 500)  = NaN;
pmPlot(abs(pmPlot) > 200)  = NaN;

Important: This is not data cleaning. The variables used later for processing are iqRaw, vqRaw, and pmRaw, not iqPlot, vqPlot, and pmPlot. This step only prevents spikes from compressing the plot axis and hiding the real waveform.

The output file from this step is: step0_raw_zoomed.png

# 6. Step 3 - Crop by Time

Each raw signal is passed through the cropByTime function. Cropping exists because real motor tests often include startup, shutdown, or unused waiting time. Those regions can contain data that does not represent the actual efficiency sweep. In this script version, startTimeSec = 0 and endTimeSec = inf, so the full time range is kept except for non-finite samples. The same function still allows the lab to crop later if a run has bad startup or shutdown behavior.

[tRpmRef, rpmRef] = cropByTime(tRpmRefRaw, rpmRefRaw, startTimeSec, endTimeSec);
[tRpm,    rpm]    = cropByTime(tRpmRaw,    rpmRaw,    startTimeSec, endTimeSec);

[tIqRef, iqRef] = cropByTime(tIqRefRaw, iqRefRaw, startTimeSec, endTimeSec);
[tIq,    iq]    = cropByTime(tIqRaw,    iqRaw,    startTimeSec, endTimeSec);

[tVq, vq] = cropByTime(tVqRaw, vqRaw, startTimeSec, endTimeSec);

[tPm, pm] = cropByTime(tPmRaw, pmRaw, startTimeSec, endTimeSec);
[tTe, te] = cropByTime(tTeRaw, teRaw, startTimeSec, endTimeSec);

function [tOut, yOut] = cropByTime(t, y, tStart, tEnd)
    if isinf(tEnd)
        mask = isfinite(t) & isfinite(y) & t >= tStart;
    else
        mask = isfinite(t) & isfinite(y) & t >= tStart & t <= tEnd;
    end
    tOut = t(mask);
    yOut = y(mask);
end

What happens to the data: only samples with finite time, finite signal value, and time inside the chosen range remain. Everything outside the time window is discarded.

# 7. Step 4 - Align All Signals to a Common Time Base

The data must be aligned because the required variables come from separate debug runs. trial01 contains speed, trial02 contains Iq, trial03 contains Vq, and trial04 contains Pm/Te. These files do not necessarily have identical time vectors or the same number of samples. Efficiency cannot be calculated unless RPM, Iq, Vq, Pm, and Te are evaluated at matching time points.

The script first finds the time interval that exists in every required signal. The common data interval starts at the latest first sample and ends at the earliest last sample.

tStart = max([safeFirst(tRpm), safeFirst(tIq), safeFirst(tVq), safeFirst(tPm), safeFirst(tTe)]);
tEnd   = min([safeLast(tRpm),  safeLast(tIq),  safeLast(tVq),  safeLast(tPm),  safeLast(tTe)]);

if tEnd <= tStart
    error('No overlapping time region across trial01-trial04.');
end

Next, the script chooses the densest available time vector as the shared time base. This preserves as much time resolution as possible.

candidates = {tRpm, tIq, tVq, tPm, tTe};
lengths = cellfun(@numel, candidates);
[~, idxDense] = max(lengths);

tBase = candidates{idxDense};
tCommon = tBase(tBase >= tStart & tBase <= tEnd);

Finally, each signal is interpolated onto tCommon.

rpmA    = interpKeepNaN(tRpm,    rpm,    tCommon);
rpmRefA = interpKeepNaN(tRpmRef, rpmRef, tCommon);

iqA     = interpKeepNaN(tIq,    iq,    tCommon);
iqRefA  = interpKeepNaN(tIqRef, iqRef, tCommon);

vqA     = interpKeepNaN(tVq, vq, tCommon);
vdA     = zeros(size(vqA));

pmA     = interpKeepNaN(tPm, pm, tCommon);
teA     = interpKeepNaN(tTe, te, tCommon);

function yq = interpKeepNaN(t, y, tq)
    good = isfinite(t) & isfinite(y);
    yq = nan(size(tq));

    if nnz(good) < 2
        return;
    end

    yq = interp1(t(good), y(good), tq, 'linear', NaN);
end

What happens to the data: each signal is resampled at the same set of time points. The interpolation is linear. Samples outside the available range become NaN. This allows one row of the final dataset to contain speed, Iq, Vq, Pm, and Te for the same moment in the sweep.

# 8. Step 5 - Force Vd to Zero

Vd is not arbitrarily ignored. In this Motor 1 efficiency test, Vd is zero in the actual Simulink model configuration. Therefore, the script represents Vd as a zero vector and uses only Vq from the Vd/Vq debug selection.

vqA     = interpKeepNaN(tVq, vq, tCommon);
vdA     = zeros(size(vqA));

What happens to the data: the measured Vq channel is preserved. Vd is set equal to zero for every aligned time sample because that matches the Simulink model behavior used for this lab.

# 9. Step 6 - Basic Sanity Filter

The basic sanity filter is the first real data-cleaning step after alignment. It removes samples that are clearly unusable before power or efficiency is calculated. This is not a physics interpretation yet; it is a guard against bad CSV values, spikes, missing samples, and values outside the possible operating range of this lab.

validBasic = isfinite(rpmA) & isfinite(iqA) & isfinite(vqA) & ...
             isfinite(pmA) & isfinite(teA) & ...
             abs(rpmA) <= maxAbsRPM & ...
             abs(iqA)  <= maxAbsIq  & ...
             abs(vqA)  <= maxAbsVq  & ...
             abs(teA)  <= maxAbsTe  & ...
             abs(pmA)  <= maxAbsPm;

The selected limits are:

maxAbsRPM = 6000;
maxAbsIq  = 8;
maxAbsVq  = 200;
maxAbsTe  = 0.30;
maxAbsPm  = 1000;

After the mask is created, the script applies it to every aligned channel so all remaining variables still refer to the same time samples.

tCommon = tCommon(validBasic);
rpmA    = rpmA(validBasic);
rpmRefA = rpmRefA(validBasic);
iqA     = iqA(validBasic);
iqRefA  = iqRefA(validBasic);
vdA     = vdA(validBasic);
vqA     = vqA(validBasic);
pmA     = pmA(validBasic);
teA     = teA(validBasic);

What this removes: non-finite samples, huge current or voltage spikes, torque values beyond the lab window, impossible RPM values, and corrupted mechanical power samples. This protects the later efficiency calculation from obvious garbage data.

# 10. Step 7 - Calculate Raw Electrical Power, Mechanical Power, and Efficiency

Once the aligned data passes the basic sanity filter, the script calculates raw electrical power, raw mechanical power, and raw efficiency.

Pelec_raw = 1.5 .* vqA .* iqA;
Pmech_raw = pmA;
eta_raw   = Pmech_raw ./ Pelec_raw;

What happens to the data: no points are removed here. This step creates new calculated arrays from the cleaned aligned signals. The values are called raw because physics-based filtering has not yet been applied.

# 11. Step 8 - Physics Filter

The physics filter removes points that passed the numerical sanity check but still do not represent useful motor efficiency data. This filter is necessary because efficiency is a ratio. When electrical power, mechanical power, or torque is very small, small noise or offsets can create ‘fake’ high-efficiency values. The low-torque region was especially problematic in Lab 1, so the torque threshold is part of the final filter.

maskFinite = isfinite(Pelec_raw) & isfinite(Pmech_raw) & isfinite(eta_raw);
maskPelec  = Pelec_raw > minPelec;
maskPmech  = Pmech_raw > minPmech;
maskTorque = teA > minTorque;
maskEta    = eta_raw > etaMin & eta_raw < etaMax;

validPhysics = maskFinite & maskPelec & maskPmech & maskTorque & maskEta;

The thresholds are:

minPelec  = 10;       % W
minPmech  = 3;        % W
minTorque = 0.045;    % N*m
etaMin    = 0.0;
etaMax    = 1.05;

| Mask | Code condition | Reason |
| --- | --- | --- |
| maskFinite | Pelec, Pmech, and eta must be finite | Rejects NaN and Inf values before final use. |
| maskPelec | Pelec_raw > 10 W | Rejects low electrical power where division can create unstable efficiency. |
| maskPmech | Pmech_raw > 3 W | Rejects near-zero mechanical output where noise dominates. |
| maskTorque | teA > 0.045 N*m | Removes the low-torque false high-efficiency band. |
| maskEta | 0 < eta_raw < 1.05 | Rejects negative and clearly non-physical efficiency values. |

What this removes: low-power idle or near-idle regions, fake high-efficiency points caused by division by small values, negative efficiency, and points above the selected efficiency limit.

# 12. Step 9 - Diagnostic Plots Without Fake Line Connections

The diagnostic plot overlays green kept points on top of the aligned raw traces. This is important because if the script plotted only the filtered points as a continuous line, MATLAB would connect separated kept samples across removed gaps and create fake ramps that were never present in the raw data.

plot(tCommon, iqA, 'r', 'LineWidth', 1.0);
scatter(tCommon(validPhysics), iqA(validPhysics), 8, 'g', 'filled');

plot(tCommon, eta_raw, 'Color', [0.5 0.5 0.5], 'LineWidth', 1.0); hold on;
scatter(tCommon(validPhysics), eta_raw(validPhysics), 8, 'g', 'filled');

The output file from this step is: step1_cleaning_overlay_no_fake_lines.png

A second diagnostic plot displays each physics mask separately so the user can see exactly which condition removed points.

nexttile; plot(tCommon, double(maskPelec), 'LineWidth', 1.0);
nexttile; plot(tCommon, double(maskPmech), 'LineWidth', 1.0);
nexttile; plot(tCommon, double(maskTorque), 'LineWidth', 1.0);
nexttile; plot(tCommon, double(maskEta), 'LineWidth', 1.0);
nexttile; plot(tCommon, double(validPhysics), 'k', 'LineWidth', 1.0);

The output file from this step is: step2_filter_masks.png

# 13. Step 10 - Apply Final Filter and Create Clean Variables

After the physics filter is finalized, the script creates a new set of clean variables. These are the variables used for the scatter plot and efficiency map.

tClean      = tCommon(validPhysics);
rpmClean    = rpmA(validPhysics);
rpmRefClean = rpmRefA(validPhysics);
iqClean     = iqA(validPhysics);
iqRefClean  = iqRefA(validPhysics);
vdClean     = vdA(validPhysics);
vqClean     = vqA(validPhysics);
pmClean     = pmA(validPhysics);
teClean     = teA(validPhysics);

Pelec = Pelec_raw(validPhysics);
Pmech = Pmech_raw(validPhysics);
eta   = eta_raw(validPhysics);

What happens to the data: every sample that fails any physics filter is removed from the final clean dataset. Every remaining row contains a synchronized and physically valid RPM, Iq, Vq, Pm, Te, electrical power, mechanical power, and efficiency value.

# 14. Step 11 - Create Valid Operating Point Scatter Plot

The first final result is a scatter plot of all valid operating points. This plot shows whether the sweep covered the intended speed and torque regions.

scatter(rpmClean, teClean, 8, eta, 'filled');
xlabel('RPM');
ylabel('Torque Te (N*m)');
title('Motor 1 Valid Operating Points Colored by Efficiency');
xlim([0 4200]);
ylim([0 0.25]);
caxis([0 1]);

The output file from this step is: step3_valid_points_efficiency_scatter.png

# 15. Step 12 - Build the Measured Efficiency Map

The measured map groups the clean operating points into speed-torque bins. This is the honest measured result before presentation interpolation.

rpmCenters    = (rpmEdges(1:end-1) + rpmEdges(2:end))/2;
torqueCenters = (torqueEdges(1:end-1) + torqueEdges(2:end))/2;

etaMap   = nan(numel(torqueCenters), numel(rpmCenters));
countMap = zeros(size(etaMap));

For every speed bin and torque bin, the script finds all clean samples inside that rectangle.

inBin = rpmClean >= rpmEdges(i) & rpmClean < rpmEdges(i+1) & ...
        teClean  >= torqueEdges(j) & teClean  < torqueEdges(j+1);

The efficiency value assigned to that map cell is the median efficiency of all samples inside the bin.

if useMedianInBins
    etaMap(j,i) = median(eta(inBin), 'omitnan');
else
    etaMap(j,i) = mean(eta(inBin), 'omitnan');
end
countMap(j,i) = sum(inBin);

Bins with too few samples are removed.

etaMap(countMap < minCountPerBin) = NaN;

Why median: median is less sensitive to remaining single-point spikes than mean. Why countMap: it shows where the map has strong data support and where it has sparse data.

Output files from this step include step4_efficiency_map_measured_bins.png and step5_count_map.png.

# 16. Step 13 - Build the Strong Interpolated Presentation Map

The interpolated map is for visualization only. It does not replace the measured binned map. The measured map is etaMap. The presentation map is etaMapSmooth.

[X, Y] = meshgrid(rpmCenters, torqueCenters);
validInterp = isfinite(etaMap);

The script builds an interpolant from only the finite measured bins.

F = scatteredInterpolant( ...
    X(validInterp), ...
    Y(validInterp), ...
    etaMap(validInterp), ...
    'natural', ...
    'nearest');

etaMapSmooth = F(X, Y);

Interpolation is restricted to regions near real measured bins. This prevents the script from filling the entire map with unsupported values.

countMask = countMap > 0;
expansionKernel = ones(9,9);
countMaskExpanded = conv2(double(countMask), expansionKernel, 'same') > 0;

etaMapSmooth(~countMaskExpanded) = NaN;

The interpolated map is then smoothed three times with a 5 by 5 weighted averaging kernel. The weight prevents NaN regions from being treated as real zero-efficiency regions.

for k = 1:3
    etaTemp = etaMapSmooth;
    etaTemp(~isfinite(etaTemp)) = 0;

    weight = double(isfinite(etaMapSmooth));
    smoothKernel = ones(5,5);

    etaFiltered = conv2(etaTemp, smoothKernel, 'same') ./ ...
                  max(conv2(weight, smoothKernel, 'same'), eps);

    etaMapSmooth(isfinite(etaMapSmooth)) = etaFiltered(isfinite(etaMapSmooth));
end

Finally, the interpolated display map is clamped to the physical display range.

etaMapSmooth(etaMapSmooth < 0) = 0;
etaMapSmooth(etaMapSmooth > 1) = 1;

Output files from this step include step6_efficiency_map_strong_interpolated.png and step7_efficiency_map_contour_strong_interpolated.png.

# 17. Step 14 - Save Final Variables and Print Summary

save(fullfile(outDir, 'motor1_efficiency_trial01_04_FINAL_data.mat'), ...
    'tCommon', 'rpmA', 'rpmRefA', 'iqA', 'iqRefA', 'vdA', 'vqA', 'pmA', 'teA', ...
    'validPhysics', 'tClean', 'rpmClean', 'rpmRefClean', 'iqClean', 'iqRefClean', ...
    'vdClean', 'vqClean', 'pmClean', 'teClean', ...
    'Pelec', 'Pmech', 'eta', 'Pelec_raw', 'Pmech_raw', 'eta_raw', ...
    'rpmEdges', 'torqueEdges', 'rpmCenters', 'torqueCenters', ...
    'etaMap', 'etaMapSmooth', 'countMap', ...
    'minPelec', 'minPmech', 'minTorque');

fprintf('Raw aligned points: %d
', numel(validPhysics));
fprintf('Kept valid points: %d
', nnz(validPhysics));
fprintf('Rejected points: %d
', numel(validPhysics) - nnz(validPhysics));
fprintf('Torque range kept: %.4f to %.4f N*m
', min(teClean), max(teClean));
fprintf('RPM range kept: %.1f to %.1f RPM
', min(rpmClean), max(rpmClean));

This summary tells the user how much data survived the processing pipeline and confirms the final RPM and torque ranges used in the map.

# 18. Helper Functions

## 18.1 readTrialSignal

Builds the file path for a specific trial folder and debug CSV file, then calls readSingleSignalCsv.

function [t, y] = readTrialSignal(baseDir, trialName, baseName)
    filePath = fullfile(baseDir, trialName, [baseName '.csv']);
    [t, y] = readSingleSignalCsv(filePath);
end

## 18.2 readSingleSignalCsv

Reads the CSV file, finds numeric columns, identifies a time column if one exists, and returns the first signal column after the time column. It also removes non-finite time or signal samples.

T = readtable(filePath, 'VariableNamingRule', 'preserve');
isNum = varfun(@isnumeric, T, 'OutputFormat', 'uniform');
Tn = T(:, isNum);

timeIdx = find(contains(lowerNames,'time') | ...
               strcmpi(names,'t') | ...
               strcmpi(names,'time_s') | ...
               strcmpi(names,'sec') | ...
               strcmpi(names,'seconds'), 1, 'first');

if isempty(timeIdx)
    t = (1:height(Tn)).';
    sigCols = 1:width(Tn);
else
    t = Tn{:, timeIdx};
    sigCols = setdiff(1:width(Tn), timeIdx, 'stable');
end

y = Tn{:, sigCols(1)};
mask = isfinite(t) & isfinite(y);
t = t(mask);
y = y(mask);

## 18.3 safeFirst and safeLast

These functions safely return the first or last time value. If a signal is empty, they return -inf or inf so the overlap calculation can still run without indexing errors.

function v = safeFirst(x)
    if isempty(x)
        v = -inf;
    else
        v = x(1);
    end
end

function v = safeLast(x)
    if isempty(x)
        v = inf;
    else
        v = x(end);
    end
end

# 19. Final Pipeline Summary

| Stage | What happens | Why it is needed |
| --- | --- | --- |
| Raw load | Reads trial01-trial04 debug CSVs | Brings required channels into MATLAB. |
| Raw plot clipping | Clips only plotting copies | Lets the user inspect raw data without huge spikes hiding the normal signal. |
| Time crop | Keeps finite samples in the selected time window | Removes startup/shutdown regions if needed. |
| Common time alignment | Interpolates separate logs onto one time vector | Allows point-by-point efficiency calculation from separately recorded variables. |
| Vd zero | Sets Vd equal to zero vector | Matches the actual Simulink model for this Lab 1 test. |
| Basic sanity filter | Removes NaN/Inf and impossible values | Prevents corrupt samples from entering calculations. |
| Power and raw efficiency | Calculates Pelec, Pmech, and eta | Creates the physical quantities needed for the efficiency map. |
| Physics filter | Keeps only adequate power, torque, and efficiency range | Removes fake high efficiency from low-power or low-torque data. |
| Clean dataset | Applies final mask to all variables | Produces synchronized valid operating points. |
| Measured bin map | Bins RPM and torque and uses median efficiency | Creates honest measured efficiency map. |
| Interpolated map | Fills and smooths only near measured bins | Creates presentation-quality visualization without replacing measured data. |

Bottom line: the processing does not simply make the plot look better. It documents a controlled path from raw trial data to synchronized, filtered, physically valid Motor 1 efficiency data.