Lab 1: Motor 1 Efficiency Mapping

Line-by-line instructions

Run Lab_1_Setup.m.

The Host model opens first.

In the Host model, click the Target model link.

Figure 1: Target model link

In the new Target model window, not the Host model, click Build, Deploy, and Start.

Figure 2: Build, Deploy, and Start in the new Target model window.

Once loaded, run the Host model.

Click the Motor1 switch to Start.

Figure 3: Motor1 switch.

Take data for the following trials using the debug selection tool:

| Trial folder | Debug selector choice |
| --- | --- |
| trial01 | Mtr1: Speed reference & Speed feedback |
| trial02 | Mtr1: Iq reference & Iq feedback |
| trial03 | Mtr1: Vd & Vq |
| trial04 | Mtr1: Pm & Te |

Figure 4: Debug Signal selection.

Once the trial ends, wait until the motor goes to near-zero speed.

Run save_debug_session.m.

Name the folder trial01, trial02, trial03, or trial04.

Once all data is saved, run data_processing.m.

View the generated figures.

If communication fails, select the correct COM port in Host Serial Setup, Host Serial Receive, and Host Serial Transmit.

Figure 5: Host serial COM-port note.