EPA Drive Cycle Testing Using a Dual-Motor PMSM Dynamometer

| Field | Description |
| --- | --- |
| Document type | Lab overview and minimum test plan |
| Platform | TI LAUNCHXL-F28069M + dual BOOSTXL-DRV8305EVM + dual PMSM dyno |
| Primary lab focus | EPA-style drive-cycle tracking, dyno loading, power/torque sign interpretation, and regen/braking identification |
| Critical correction | Positive Motor 2 Iq = Motor 2 load/dyno action; negative Motor 2 Iq = Motor 2 driving/assisting action |

Important Sign-Convention Correction In this setup, a positive Motor 2 Iq means Motor 2 is acting as the load, which is why Motor 1's mechanical power (Pm1) shows a meaningful drive-power reading. Negative Motor 2 Iq means Motor 2 is acting as the driver or assist motor, which is why Motor 2's mechanical power (Pm2) shows the meaningful drive-power reading.

# 1. Purpose The purpose of Lab 2 is to evaluate the dynamic behavior of the dual-motor PMSM dynamometer during an EPA-style drive cycle. Lab 1 focuses on steady-state efficiency-map coverage. Lab 2 instead asks how the coupled motor system behaves while tracking a changing speed command. The main outputs are speed tracking, Motor 2 load direction, Motor 1 and Motor 2 mechanical power, Motor 1 and Motor 2 torque, and an estimate of where energy is consumed or recovered.

# 2. Scope of the Experiment

The lab uses a single drive-cycle command and repeats it several times, changing only the selected debug signal pair each time. This produces multiple comparable logs of the same physical event. Students then align and interpret the traces to determine which motor is driving, which motor is loading, whether speed tracking is acceptable, and, where possible, regen or braking behavior occurs.

Core Rule Every debug trial must use the same EPA-style drive-cycle command. Do not change speed profile, load profile, timing, controller gains, communication settings, or hardware wiring between trials.

# 3. Learning Objectives

- Verify that Motor 1 follows the EPA-style speed command.

- Quantify speed tracking error and identify where tracking is weakest.

- Use Motor 2 Iq sign to determine when Motor 2 is loading versus driving.

- Explain why Motor 1 Pm is meaningful during positive Motor 2 Iq regions.

- Explain why Motor 2 Pm is meaningful during negative Motor 2 Iq regions.

- Compare the torque behavior of Motor 1 and Motor 2 during the load and assist regions.

- Compare the mechanical power behavior of Motor 1 and Motor 2 during the cycle.

- Identify possible braking or regen regions from Iq, torque, and power direction.

- State the efficiency assumption used for Motor 2 and its limitations.

# 4. Hardware and Software Context

The lab is built around a TI C2000 motor-control workflow. The expected configuration is a LaunchPad controller, two inverter BoosterPacks, two PMSM machines, encoder feedback, a 24 V supply, and a Simulink host/target workflow.

| Category | Requirement | Lab 2 Relevance |
| --- | --- | --- |
| Controller | TI LAUNCHXL-F28069M | Runs the C2000 motor-control target code and communicates with the host model. |
| Inverters | 2 x BOOSTXL-DRV8305EVM | One inverter drives Motor 1, and another inverter controls Motor 2 as the dyno/load or assist motor. |
| Machines | 2 x PMSM motors | Mechanically coupled for shaft power transfer and dyno testing. |
| Supply | 24 V DC supply, at least 5 A recommended | Provides inverter-bus power to both power stages. |
| Feedback | Encoder/QEP feedback for both motors | Required for speed feedback, FOC operation, and coupling verification. |
| Software | MATLAB/Simulink, CCS, C2000Ware/controlSUITE as required by project | Used to build, deploy, command, and log the dyno system. |

# 5. System-Level Behavior

The dyno system should be interpreted as a coupled two-machine shaft. At any instant, one motor may be the primary source of mechanical power while the other motor applies a load or absorbs energy. The sign convention used in this lab determines which motor mechanical-power trace is meaningful in each region.

| Operating Region | Motor 2 Iq Sign | Motor 2 Role | Primary Meaningful Pm Trace | Interpretation |
| --- | --- | --- | --- | --- |
| Motor 1 drive/dyno loading | Positive | Load/dyno motor | Motor 1 Pm | Motor 1 is doing useful shaft work against the load of Motor 2. |
| Motor 2 drive/assist | Negative | Driver / assisting motor | Motor 2 Pm | Motor 2 is supplying useful shaft work. |
| Transition / no load | Near zero | Minimal load or drive action | Neither trace is strongly reliable by itself | Use caution; power may be small, noisy, or sign-convention dominated. |

Practical Interpretation Do not interpret Pm1 and Pm2 as equally valid over the entire drive cycle. Pm1 is the main useful mechanical-power trace when Motor 2 Iq is positive. Pm2 is the main useful mechanical-power trace when Motor 2 Iq is negative.

# 6. Required Debug Trials

The following seven trials are the minimum complete set. Each trial uses the same EPA-style drive-cycle command. Only the debug signal selection changes.

| Trial | Folder Name | Debug Signal Selection | Why It Is Needed |
| --- | --- | --- | --- |
| 01 | trial01_speed_tracking | Mtr1: Speed ref & Speed feedback | Shows whether Motor 1 follows the EPA speed command. |
| 02 | trial02_mtr2_iq | Mtr2: Iq ref & Iq feedback | Determines whether Motor 2 is loading or driving based on the Iq sign. |
| 03 | trial03_mtr1_pm_te | Mtr1: Pm & Te | Shows Motor 1 mechanical power and torque. |
| 04 | trial04_mtr2_pm_te | Mtr2: Pm & Te | Shows Motor 2 mechanical power and torque. |
| 05 | trial05_both_pm | Mtr1&Mtr2: Pm | Directly compares both motor mechanical-power traces. |
| 06 | trial06_both_te | Mtr1&Mtr2: Te | Directly compares both motor torque traces. |
| 07 | trial07_mtr1ref_mtr2speed | Mtr1 Speed ref & Mtr2 Speed fb | Confirms that Motor 2 is mechanically coupled and follows the same cycle. |

# 7. Pre-Test Checklist

| Check Item | Requirement |
| --- | --- |
| Power supply | 24 V supply connected to the correct PVDD/GND terminals. Verify polarity with a DMM before enabling power. |
| Board stack | LaunchPad and both BoosterPacks are mounted correctly with no offset pins. |
| Motor phase wiring | Phase order is consistent for each motor. Record wiring before running. |
| Encoder wiring | Motor 1 and Motor 2 encoder feedback are connected to correct QEP inputs. |
| Mechanical coupling | The shaft coupling is tight and guarded. No visible wobble or slip. |
| Host communication | The host model communicates with the target and receives live feedback. |
| Target code | The target model is built, deployed, and running. |
| Debug selection | Correct debug pair selected before each trial. |
| Data save path | Folder naming is ready before starting the first run. |

# 8. Safety and Handling Notes

- Verify 24 V polarity before powering the inverters. Incorrect polarity can permanently damage the boards.

- Keep hands, wires, and loose objects away from the rotating shaft coupling during operation.

- Use conservative load/current limits until the dyno behavior is confirmed.

- Do not change wiring while the 24 V supply is energized.

- If the motor vibrates, spins unexpectedly, loses encoder feedback, or produces abnormal noise, stop the run and turn off the system before troubleshooting.

- Allow power components to cool if repeated tests are run back-to-back.

# 9. Run Procedure

1. Open the host model and confirm the target is connected.

2. Select the EPA-style drive-cycle command input.

3. Select the debug signal pair for the current trial.

4. Run the drive cycle without changing the speed profile, load profile, or controller settings.

5. Stop the run only after the full cycle is complete unless unsafe behavior occurs.

6. Save the data immediately after the run.

7. Rename the saved folder using the required folder name for that trial.

8. Proceed to the next trial and repeat the process using the same drive-cycle command.

# 10. Analysis Methodology

## 10.1 Speed Tracking

Use trial01_speed_tracking to compare Motor 1 speed reference against Motor 1 speed feedback. The feedback should follow the EPA drive-cycle shape. The largest error will usually occur during acceleration and deceleration because these regions require the fastest torque and speed response.

speed_error = speed_reference - speed_feedback;
max_abs_error = max(abs(speed_error));
mean_abs_error = mean(abs(speed_error));

| Observation | Likely Meaning |
| --- | --- |
| Feedback follows the command shape | The drive-cycle command path, speed loop, encoder feedback, and SCI timing are likely valid. |
| Large acceleration error | Motor torque/current limit, load demand, or speed-loop response may be limiting tracking. |
| Large deceleration error | Braking/load-transition dynamics or the sign convention may dominate. |
| Feedback does not follow the EPA shape. | Wrong command path, failed run, communication issue, or mechanical/electrical setup fault. |

## 10.2 Motor 2 Iq Sign Interpretation

Use trial02_mtr2_iq to determine which motor is acting as the source of mechanical power and which motor is acting as the load. This lab uses the corrected sign convention below.

| Motor 2 Iq | Motor 2 Role | Meaning for Pm Analysis |
| --- | --- | --- |
| Positive | Load/dyno motor | Motor 1 is the useful drive source; analyze Motor 1 Pm. |
| Negative | Driver / assisting motor | Motor 2 is the useful drive source; analyze Motor 2 Pm. |
| Near zero | Transition or no meaningful load | Use caution; neither Pm trace may be physically meaningful by itself. |

validMotor1Drive = mtr2Iq > 0;
validMotor2Drive = mtr2Iq < 0;
nearZeroIq = abs(mtr2Iq) < iqThreshold;

## 10.3 Motor 1 Power and Torque

Use trial03_mtr1_pm_te. Motor 1 Pm and Te are most meaningful during positive Motor 2 Iq regions. In those regions, Motor 2 acts as the load, and Motor 1 performs useful shaft work.

Pm1_valid = Pm1;
Te1_valid = Te1;
Pm1_valid(~validMotor1Drive) = NaN;
Te1_valid(~validMotor1Drive) = NaN;

## 10.4 Motor 2 Power and Torque

Use trial04_mtr2_pm_te. Motor 2 Pm and Te are most meaningful during negative Motor 2 Iq regions. In those regions, Motor 2 acts as the driver or an assisting motor.

Pm2_valid = Pm2;
Te2_valid = Te2;
Pm2_valid(~validMotor2Drive) = NaN;
Te2_valid(~validMotor2Drive) = NaN;

## 10.5 Direct Power Comparison

Use trial05_both_pm to compare Motor 1 and Motor 2 mechanical power. The two traces should be interpreted with the Iq sign mask. Pm1 is the primary useful reading when Motor 2 Iq is positive. Pm2 is the primary useful reading when Motor 2 Iq is negative.

## 10.6 Direct Torque Comparison

Use trial06_both_te to compare Motor 1 and Motor 2 torque. During loading, the torque directions should generally be opposed. During transition, braking, or assist regions, signs may reverse or pass through zero.

## 10.7 Mechanical Coupling Check

Use trial07_mtr1ref_mtr2speed to confirm Motor 2 speed follows the same mechanical motion as Motor 1. If Motor 2 speed does not follow the EPA cycle shape, then the power and torque interpretation is not trustworthy.

# 11. Required Plots

| Plot | Signals | Purpose |
| --- | --- | --- |
| 1 - EPA Speed Tracking | Motor 1 speed reference vs Motor 1 speed feedback | Shows whether the drive motor tracks the EPA command. |
| 2 - Motor 2 Iq | Motor 2 Iq reference vs Motor 2 Iq feedback | Shows load, drive/assist, and transition regions. |
| 3 - Motor 1 Mechanical Output | Motor 1 Pm and Motor 1 Te | Shows Motor 1 useful output during positive Motor 2 Iq loading regions. |
| 4 - Motor 2 Mechanical Output | Motor 2 Pm and Motor 2 Te | Shows Motor 2 useful output during negative Motor 2 Iq driving regions. |
| 5 - Both Motor Powers | Motor 1 Pm vs Motor 2 Pm | Compares mechanical power transfer across the coupled shaft. |
| 6 - Both Motor Torques | Motor 1 Te vs Motor 2 Te | Compares torque direction and load opposition. |
| 7 - Coupling Check | Motor 1 speed reference vs Motor 2 speed feedback | Confirms Motor 2 follows the coupled shaft motion. |

# 12. Approximate Energy Calculations

Energy can be estimated by integrating mechanical power over time. This lab does not require perfect energy accounting; it requires identifying where mechanical energy is supplied, absorbed, or possibly recovered.

E_mtr1_drive = trapz(t(validMotor1Drive), Pm1(validMotor1Drive));
E_mtr2_drive = trapz(t(validMotor2Drive), Pm2(validMotor2Drive));

| Quantity | Interpretation |
| --- | --- |
| E_mtr1_drive | Approximate mechanical energy supplied by Motor 1 during positive Motor 2 Iq regions. |
| E_mtr2_drive | Approximate mechanical energy supplied by Motor 2 during negative Motor 2 Iq regions. |
| Power peaks | Regions of highest instantaneous shaft power demand. |
| Sign changes | Potential braking, regen, or transition regions depending on the torque and speed sign. |

# 13. Motor 2 Efficiency Assumption

Motor 2's electrical efficiency is not directly measured in this lab. Therefore, Motor 2 efficiency is approximated using the Motor 1 efficiency map from Lab 1. This approximation is acceptable for a behavioral drive-cycle lab because the two motors and inverter boards are treated as matched hardware. However, it is not a final calibrated efficiency measurement.

| Statement | Required Student Treatment |
| --- | --- |
| Motor 2 efficiency is estimated. | Students must explicitly state that Motor 2 efficiency is not directly measured. |
| The estimate uses the Motor 1 efficiency map. | Students must explain that the approximation is used because both machines and boards are identical. |
| The estimate is temporary. | Students must not present Motor 2 estimated efficiency as certified or calibrated data. |
| Sign convention affects interpretation. | Students should apply the positive/negative Motor 2 Iq rule before using Pm data. |

# 14. Discussion Questions

- Does Motor 1 follow the EPA speed command? Include speed reference and speed feedback evidence.

- Where is the largest Motor 1 speed tracking error?

- Does tracking error increase more during acceleration, deceleration, or load transition?

- When is Motor 2 acting as the load according to the corrected Iq sign convention?

- When is Motor 2 acting as the driver or assist motor?

- Why does Motor 1 Pm show the useful reading when Motor 2 Iq is positive?

- Why does Motor 2 Pm show the useful reading when Motor 2 Iq is negative?

- How do Motor 1 and Motor 2 torques compare during loading?

- How do Motor 1 and Motor 2 mechanical powers compare over the cycle?

- Where does the system demand the most mechanical power?

- Where might braking or regen behavior occur?

- Why is Motor 2 efficiency estimated from Motor 1 efficiency?

- What are the limits of this efficiency assumption?

# 15. Deliverables

- All seven required plots with readable titles, axis labels, units, and legends.

- Speed tracking error calculation, including maximum absolute error and mean absolute error.

- Motor 2 Iq sign-region labels showing positive-Iq loading and negative-Iq driving/assist regions.

- Masked Motor 1 Pm plot showing Motor 1 useful drive regions only.

- Masked Motor 2 Pm plot showing Motor 2 useful drive regions only.

- Approximate mechanical energy estimate using area under the relevant power curves.

- Written answers to the discussion questions.

- Explicit statement that Motor 2 efficiency is estimated, not directly measured.

- Conclusion paragraph summarizing whether the dyno test was valid and what the data shows.

# 16. Recommended Report Structure

| Section | Content |
| --- | --- |
| Introduction | State the purpose of Lab 2 and describe the dual-motor dyno system. |
| Methods | Describe the EPA drive-cycle run procedure and the seven debug trials. |
| Sign Convention | Explain the corrected Motor 2 Iq sign convention and how it affects Pm interpretation. |
| Results | Include required plots and speed tracking error. |
| Analysis | Discuss loading, driving/assist, torque comparison, power comparison, and energy estimates. |
| Efficiency Assumption | Explain why Motor 2 efficiency is estimated from Motor 1 map data. |
| Conclusion | Summarize whether the run was valid, what the system did, and what should be improved. |

# Appendix A - MATLAB-Style Analysis Skeleton

% Lab 2 analysis skeleton
% Assumes synchronized vectors: t, speedRef1, speedFb1, mtr2Iq, Pm1, Pm2, Te1, Te2

% 1. Speed tracking
speedError = speedRef1 - speedFb1;
maxAbsSpeedError = max(abs(speedError));
meanAbsSpeedError = mean(abs(speedError));

% 2. Corrected sign convention
validMotor1Drive = mtr2Iq > 0;   % Motor 2 is load/dyno; Motor 1 Pm is meaningful
validMotor2Drive = mtr2Iq < 0;   % Motor 2 is driver/assist; Motor 2 Pm is meaningful
nearZeroIq = abs(mtr2Iq) < 0.05; % adjust threshold as appropriate

% 3. Mask traces for physically meaningful interpretation
Pm1_valid = Pm1;
Pm2_valid = Pm2;
Te1_valid = Te1;
Te2_valid = Te2;

Pm1_valid(~validMotor1Drive) = NaN;
Te1_valid(~validMotor1Drive) = NaN;
Pm2_valid(~validMotor2Drive) = NaN;
Te2_valid(~validMotor2Drive) = NaN;

% 4. Approximate energy estimates
E_mtr1_drive = trapz(t(validMotor1Drive), Pm1(validMotor1Drive));
E_mtr2_drive = trapz(t(validMotor2Drive), Pm2(validMotor2Drive));

% 5. Basic plots
figure; plot(t, speedRef1, t, speedFb1); grid on;
xlabel('Time (s)'); ylabel('Speed (RPM)');
title('Motor 1 EPA Speed Tracking'); legend('Speed Ref','Speed Feedback');

figure; plot(t, mtr2Iq); grid on;
xlabel('Time (s)'); ylabel('Motor 2 Iq (A)');
title('Motor 2 Iq Sign Convention: Positive = Load, Negative = Driver');

figure; plot(t, Pm1_valid); grid on;
xlabel('Time (s)'); ylabel('Motor 1 Pm');
title('Motor 1 Mechanical Power During Positive Motor 2 Iq Loading Regions');

figure; plot(t, Pm2_valid); grid on;
xlabel('Time (s)'); ylabel('Motor 2 Pm');
title('Motor 2 Mechanical Power During Negative Motor 2 Iq Driving Regions');

# Appendix B - Instructor Check Rubric

| Item | Full Credit Criteria |
| --- | --- |
| Same EPA command used for all trials | Student states that all seven trials used the same speed/load/timing conditions. |
| Correct Iq sign convention | Student correctly states positive Motor 2 Iq = Motor 2 load and negative Motor 2 Iq = Motor 2 driver/assist. |
| Correct Pm interpretation | Student analyzes Pm1 in positive-Iq regions and Pm2 in negative-Iq regions. |
| Speed tracking | The student provides a reference vs feedback plot and error calculation. |
| Coupling check | Student verifies Motor 2 speed follows the cycle. |
| Torque comparison | Student discusses torque opposition and sign changes. |
| Energy estimate | Student integrates appropriate power traces or qualitatively identifies energy regions. |
| Efficiency limitation | The student says that Motor 2 efficiency is estimated rather than directly measured. |

# Appendix C - Common Student Mistakes to Watch For

| Mistake | Correction |
| --- | --- |
| Using the old sign convention | Students may incorrectly write that negative Motor 2 Iq is the load region. In this lab setup, positive Motor 2 Iq corresponds to the Motor 2 load/dyno region. |
| Interpreting both Pm traces over the full cycle. | Students should mask the data. Pm1 is meaningful in positive-Iq loading regions; Pm2 is meaningful in negative-Iq driving/assist regions. |
| Changing the command between debug trials | If the command changes, the seven debug logs are not comparable. Require the same EPA drive-cycle input for every trial. |
| Skipping the coupling check | If the 2-speed motor does not follow the cycle, the torque and power interpretations are not trustworthy. |
| Treating estimated efficiency as measured efficiency | Motor 2 efficiency is estimated from Motor 1 map data and should be described as an approximation only. |