Lab 3 Basic Build Guide

Speedgoat CAN Control for the Dual-Motor PMSM Dyno

what to build, wire, test, and prove

One-sentence goal Replace the old MATLAB/SCI command and debug path with a Speedgoat-to-C2000 CAN path so Lab 3 can command the same dual-motor dyno system used in Lab 2.

# 1. What Lab 3 Is

Lab 3 is the communication and integration lab. Lab 2 proved the dual-motor dyno behavior using the existing host model and SCI serial communication. Lab 3 keeps the motor-control idea but changes the communication path: Speedgoat becomes the real-time controller/host, and the LAUNCHXL-F28069M receives commands and sends telemetry over CAN.

| Old Lab 2 Path | New Lab 3 Path |
| --- | --- |
| MATLAB host sends RPM/Iq/debug over SCI | Speedgoat sends RPM/Iq/mode/debug over CAN |
| C2000 target receives SCI command packet | C2000 target receives CAN command frames |
| C2000 sends debug values back over SCI | C2000 sends speed, Iq, torque, and power back over CAN |
| Data is logged on MATLAB host scopes | Data is logged on Speedgoat scopes/logging |

# 2. Must-Know Sign Convention

Use this sign rule in all Lab 3 analysis Positive Motor 2 Iq = Motor 2 acts as the load/dyno, so Motor 1 Pm is the meaningful drive-power reading. Negative Motor 2 Iq = Motor 2 acts as the driver/assist motor, so Motor 2 Pm is the meaningful drive-power reading.

| Motor 2 Iq | Motor 2 Role | Main Useful Power Signal |
| --- | --- | --- |
| Positive | Load / dyno | Motor 1 Pm |
| Negative | Driver / assist motor | Motor 2 Pm |
| Near zero | Transition / little useful loading | Use caution |

# 3. What You Need

| Item | Purpose |
| --- | --- |
| Speedgoat real-time target | Runs the Lab 3 command, logging, and CAN receive model |
| LAUNCHXL-F28069M LaunchPad | C2000 motor-control target receiving CAN commands |
| Two BOOSTXL-DRV8305EVM boards | Motor inverters for Motor 1 and Motor 2 |
| Two PMSM motors mechanically coupled | Dual-motor dynamometer |
| 24 V DC supply | Power for inverter boards |
| CAN connection between Speedgoat and C2000 | Command and telemetry bus |
| 120 ohm CAN termination | Stable CAN physical layer |
| Simulink + C2000 support package | Build/deploy C2000 target model |
| Speedgoat/Simulink Real-Time CAN blocks | Build Speedgoat host model |

# 4. Correct Simulink Block Direction

Important On the C2000 target model, use the C2000/C28x CAN blocks for the F28069M target. Do not use only the generic desktop CAN blocks inside the C2000 code-generation model. The Speedgoat side should be able to use the Speedgoat/Simulink Real-Time CAN interface blocks.

| Model Side | Use This | For This |
| --- | --- | --- |
| Speedgoat/Host model | CAN Pack / CAN Transmit / CAN Receive / CAN Unpack or Speedgoat CAN equivalent bblocks | Command generation, CAN TX/RX, logging |
| C2000 F28069M target model | C28x CAN Receive and C28x CAN Transmit | Receiving commands and transmitting telemetry |
| C2000 hardware config | CAN_A / eCAN | The CAN peripheral used by the LaunchPad target |
| Optional development path | SCI left in model as fallback | Debugging if CAN is not working yet |

# 5. Final Architecture

Speedgoat EPA command and load schedule -> CAN command frames -> LAUNCHXL-F28069M target -> PMSM dyno -> CAN telemetry frames -> Speedgoat logging and plots

| CAN Direction | Data Moving Across Bus |
| --- | --- |
| Speedgoat to C2000 | Enable, mode, trial number, Motor 1 speed reference, Motor 2 Iq reference |
| C2000 to Speedgoat | Speed feedback, Motor 2 Iq feedback, Motor 1 Pm/Te, Motor 2 Pm/Te, status/faults |

# 6. Simple CAN Message Plan

Start with a small message set. Do not try to send the entire lab in one CAN frame. Use fixed 11-bit CAN IDs and fixed scaling so both models decode the data the same way.

| CAN ID | Name | Direction | Contains |
| --- | --- | --- | --- |
| 0x100 | Command_Status | Speedgoat -> C2000 | Enable, mode, debug/trial number, command counter |
| 0x101 | Speed_Command | Speedgoat -> C2000 | Motor 1 speed reference |
| 0x102 | Iq_Command | Speedgoat -> C2000 | Motor 2 Iq/load reference |
| 0x200 | Target_Status | C2000 -> Speedgoat | Enable state, mode, fault flags, command counter echo |
| 0x201 | Speed_Telemetry | C2000 -> Speedgoat | Speed reference and speed feedback |
| 0x202 | Iq_Telemetry | C2000 -> Speedgoat | Motor 2 Iq reference and feedback |
| 0x203 | Mtr1_Telemetry | C2000 -> Speedgoat | Motor 1 Pm and Te |
| 0x204 | Mtr2_Telemetry | C2000 -> Speedgoat | Motor 2 Pm and Te |

Recommended scaling Speed: int16, 1 RPM/count for first build. Iq: int16, 0.001 A/count or 0.01 A/count. Power/Torque: choose a simple fixed scale and document it in both models. Always include a command counter or heartbeat so you can tell if messages are fresh.

# 7. Build Checklist

| Step | Do This | Done? |
| --- | --- | --- |
| 1 | Identify the CAN physical pins/connector on the LAUNCHXL-F28069M setup. |  |
| 2 | Wire CANH, CANL, and required ground/reference between Speedgoat and C2000 side. |  |
| 3 | Add 120 ohm termination at the two ends of the CAN bus. |  |
| 4 | Set both sides to the same CAN bit rate, such as 500 kbps or 1 Mbps. |  |
| 5 | Add C28x CAN Receive blocks to the C2000 target model. |  |
| 6 | Decode speed/Iq/enable commands and route them to the existing motor-control inputs. |  |
| 7 | Add C28x CAN Transmit blocks for telemetry. |  |
| 8 | Build a Speedgoat model that sends command frames and receives telemetry frames. |  |
| 9 | Run CAN echo test with motors disabled. |  |
| 10 | Run low-speed command test with small values. |  |
| 11 | Run positive Motor 2 Iq load test and verify Motor 1 Pm appears. |  |
| 12 | Run negative Motor 2 Iq assist test and verify Motor 2 Pm appears. |  |
| 13 | Run the final Lab 3 drive-cycle demonstration. |  |

# 8. Safety and Timeout Rules

| Rule | Reason |
| --- | --- |
| Motors stay disabled until CAN echo works. | Prevents accidental motion from bad decoding. |
| If CAN command timeout occurs, disable PWM or force safe zero command. | Prevents stale command runaway. |
| Start with low speed and low Iq. | Limits torque and mechanical risk. |
| Use a command counter/heartbeat. | Confirms commands are updating. |
| Keep SCI fallback during early testing. | Provides a recovery path if CAN path fails. |
| Verify sign convention before EPA cycle. | Prevents interpreting load/drive backwards. |

# 9. Bring-Up Sequence

| Phase | Test | Pass Condition |
| --- | --- | --- |
| Phase 1 | CAN echo test, motors disabled | Speedgoat sends counter; C2000 echoes same counter. |
| Phase 2 | Decode test, motors disabled | C2000 decodes enable, speed, and Iq correctly. |
| Phase 3 | Low-speed Motor 1 test | Motor 1 follows a small speed command. |
| Phase 4 | Positive Iq load test | Motor 2 loads shaft; Motor 1 Pm becomes meaningful. |
| Phase 5 | Negative Iq assist test | Motor 2 drives/assists; Motor 2 Pm becomes meaningful. |
| Phase 6 | Lab 3 full run | Speedgoat commands drive cycle and logs all required telemetry. |

# 10. What Students Need to Submit

| Deliverable | What It Should Show |
| --- | --- |
| CAN architecture screenshot | Speedgoat command path and C2000 CAN receive/transmit path |
| CAN message table | IDs, directions, signals, scaling, and update rates |
| Echo test result | Speedgoat-C2000-Speedgoat communication works |
| Speed tracking plot | Motor 1 speed reference vs speed feedback |
| Motor 2 Iq plot | Iq reference and feedback |
| Power/torque plots | Motor 1 and Motor 2 Pm/Te |
| Sign-convention explanation | Positive Iq = Motor 2 load; negative Iq = Motor 2 driver |
| Short conclusion | Whether Lab 3 successfully replaced SCI with CAN |

# 11. Minimum Successful Lab 3

Minimum pass condition A student should be able to say: Speedgoat sent speed and Iq commands over CAN, the LAUNCHXL-F28069M target received and applied them, the dual-motor dyno responded, and C2000 telemetry returned over CAN to Speedgoat for logging.

# 12. Five-Line Summary

- Lab 3 replaces SCI with CAN.

- Speedgoat becomes the real-time host/controller.

- The C2000 F28069M target uses C28x CAN Receive/Transmit on CAN_A.

- Start with echo test, then command test, then low-speed motor test, then full dyno test.

- Use the corrected sign convention: positive Motor 2 Iq loads Motor 1; negative Motor 2 Iq means Motor 2 is driving.

# Appendix. Reference Screenshots From Current Model

Figure A-1. Current host-side SCI transmit/receive pathway that Lab 3 replaces with CAN command and telemetry paths.

Figure A-2. Current C2000 target SCI transmit block area, indicating where CAN telemetry transmission should be added for Lab 3.