Lab 3 Overview

Implementing the Dual-Motor PMSM Dyno EPA Drive-Cycle Lab Through Speedgoat CAN

LAUNCHXL-F28069M / C2000 Target Model Communication Conversion

Lab 3 purpose Convert the Lab 2 SCI/host-model control path into a CAN-based real-time control and telemetry path so Speedgoat can command the same dual-motor dyno behavior over CAN. The motor-control algorithm should remain functionally equivalent; the communication transport changes from SCI serial packets to structured CAN messages.

| Document item | Description |
| --- | --- |
| Primary hardware | Speedgoat real-time machine, TI LAUNCHXL-F28069M LaunchPad, dual BOOSTXL-DRV8305 inverter stack, two coupled PMSMs |
| Primary software | Simulink Real-Time/Speedgoat model, C2000 target model, C28x CAN blocks, optional DBC/message dictionary |
| Main conversion | Replace SCI transmit/receive command and debug pathways with CAN command and telemetry frames |
| Lab continuity | Preserve Lab 2 corrected sign convention: positive Motor 2 Iq = Motor 2 load; negative Motor 2 Iq = Motor 2 driver/assist |
| Recommended first target | CAN echo test, then command-only decode, then low-speed dyno operation, then full EPA drive-cycle |

# 1. Executive Summary

Lab 3 extends Lab 2 by moving the dyno command and data-collection interface from a MATLAB host/SCI serial workflow to a Speedgoat/CAN workflow. The existing Lab 2 experiment already defines the required dyno behavior: Motor 1 follows an EPA-style speed command while Motor 2 applies load or drive/assist torque. Lab 3 keeps that control objective, but changes how commands and telemetry move between the real-time host and the C2000 target.

The current host model uses SCI paths for command transmission and debug-signal reception. In Lab 3, those SCI pathways should be replaced or bypassed with CAN pathways. Speedgoat becomes the real-time supervisory controller. The F28069M LaunchPad remains the embedded motor-control endpoint. The CAN bus becomes the deterministic command/status link between them.

Core Lab 3 concept Do not redesign the motor-control system first. Replace the communication layer first. Treat the SCI serial packet structure as the functional prototype, then re-create the same command and telemetry behavior with CAN frames.

# 2. Relationship Between Lab 2 and Lab 3

| Item | Lab 2 | Lab 3 |
| --- | --- | --- |
| Real-time command source | MATLAB/Simulink host model over SCI | Speedgoat real-time model over CAN |
| Embedded target | LAUNCHXL-F28069M running C2000 PMSM FOC target model | Same target model, modified for CAN command and telemetry |
| Drive-cycle source | EPA-style speed command from host workspace or model | EPA-style speed command generated or replayed in Speedgoat |
| Load command | Motor 2 Iq/load command from host model | Motor 2 Iq/load command transmitted over CAN |
| Debug data | SCI debug_1/debug_2 values selected by Debug_signals | CAN telemetry frames, initially debug_1/debug_2, eventually all required Lab 2 signals |
| Main student objective | Interpret speed tracking, load behavior, Pm, Te, and regen/assist regions | Implement and validate the same experiment through a production-style CAN interface |

Lab 3 can be implemented in two maturity levels. The first level is an exact functional copy of Lab 2: Speedgoat sends the same command fields, including the debug-signal selection, and the C2000 returns two selected debug values. The second level is the preferred final architecture: the C2000 continuously streams all required Lab 2 telemetry over CAN so Speedgoat can log the full drive cycle in one run.

# 3. Existing SCI-Based Architecture

The current model has a host-side transmit path and receive path. The transmit path packages speed reference, Iq reference, motor start/stop, motor selection/change bit, and debug-signal selection. The receive path unpacks debug data and converts it back into engineering units for display as debug_1 and debug_2.

MATLAB/Simulink Host Model
        |
        | SCI serial TX command packet
        v
C2000 F28069M Target Model
        |
        | Motor 1 speed command
        | Motor 2 Iq/load command
        v
Dual PMSM Dyno
        |
        | SCI serial RX debug packet
        v
Host debug_1 / debug_2 scopes

| SCI path element | Existing role | CAN replacement |
| --- | --- | --- |
| SCI_TX block | Sends packed command data to the target | C28x CAN Receive on target; Speedgoat CAN Transmit on host |
| Host Serial Receive block | Receives target debug data | Speedgoat CAN Receive block |
| Parse block | Combines enable, motor select, debug select, RPM, and Iq into packet data | CAN command-frame packer or message dictionary/DBC encoder |
| Display_Value / Convert_to_SI_Units | Unpacks returned debug values and converts to physical units | CAN telemetry unpacker and engineering-unit scaling |
| Debug_signals constant | Selects which two target variables are returned | Either preserve as CAN field or replace with continuous telemetry frames |

# 4. Target CAN-Based Architecture

The target architecture places Speedgoat in the role of the supervisory real-time controller. The C2000 target still closes the fast motor-control loops. CAN messages carry command references from Speedgoat to the C2000 and telemetry/status data from the C2000 back to Speedgoat.

Speedgoat Real-Time Model
        |
        | CAN command frames
        v
LAUNCHXL-F28069M / C2000 Target Model
        |
        | Existing PMSM FOC + dyno control logic
        v
Dual-Motor PMSM Dyno
        |
        | CAN telemetry/debug frames
        v
Speedgoat scopes, logging, and Lab 3 analysis

Implementation principle The CAN conversion should preserve the same command semantics as the SCI model: Motor 1 speed reference, Motor 2 Iq/load reference, enable/start-stop state, motor selection/change state, and debug or telemetry selection.

# 5. Correct C2000 Block Family and Hardware Target

For the C2000 target model, the CAN pathway should use the C2000/C28x CAN blocks appropriate for the LAUNCHXL-F28069M target. The generic Vehicle Network Toolbox CAN blocks are useful on the host/desktop/Speedgoat side, but the embedded C2000 target requires code-generation-compatible C28x CAN blocks.

| Function | Recommended C2000-side implementation |
| --- | --- |
| Receive commands from Speedgoat | C28x CAN Receive block |
| Transmit telemetry to Speedgoat | C28x CAN Transmit block |
| CAN peripheral | CAN_A / eCAN module on F28069M |
| Hardware board setting | TI Piccolo F28069M LaunchPad / LAUNCHXL-F28069M |
| Initial bit rate | Start at 500 kbps for bring-up or 1 Mbps if wiring is short and clean |
| Frame format | Standard 11-bit CAN IDs for the first implementation |
| Payload limit | Classic CAN 8-byte data payload per frame |

The user-provided MathWorks CAN overview shows the general host-side workflow: CAN Configuration, CAN Pack, CAN Transmit, CAN Receive, and CAN Unpack blocks move structured signals over CAN. It also notes that CAN Receive can generate a function-call trigger when a new message arrives. In Lab 3, that same concept should be applied carefully: Speedgoat can pack/transmit command frames and receive/unpack telemetry, while the F28069M target uses C28x CAN blocks for embedded code generation.

# 6. Required Hardware Checks

| Check | Requirement | Reason |
| --- | --- | --- |
| CAN physical pins | Identify CANH, CANL, and ground reference for the F28069M CAN path | Prevents wiring the wrong header or bypassing the transceiver incorrectly |
| Transceiver path | Confirm whether the LaunchPad onboard CAN transceiver is used or whether an external transceiver is required | C2000 CAN pins are logic-level; the physical CAN bus needs a transceiver |
| Termination | Use 120 ohm termination at both ends of the bus | Prevents reflections and corrupted frames |
| Ground reference | Confirm Speedgoat and target CAN reference/ground arrangement | Improves noise immunity and avoids common-mode issues |
| Baud rate | Set the same bit rate on Speedgoat and C2000 | Mismatched baud rate prevents communication |
| CAN IDs | Use fixed 11-bit IDs during bring-up | Simplifies filtering and debugging |
| Power safety | Keep PWM disabled during initial CAN decode tests | Prevents accidental motor motion |

# 7. CAN Message Strategy

Classic CAN frames only carry 8 data bytes, so Lab 3 should not try to send the entire dyno command or telemetry set in one message. Use a small set of fixed CAN IDs grouped by purpose. This makes the implementation easier to test, easier to decode, and easier to expand.

## 7.1 Speedgoat to C2000 Command Frames

| CAN ID | Frame name | Direction | Purpose |
| --- | --- | --- | --- |
| 0x100 | Dyno_Command_1 | Speedgoat -> C2000 | Enable, motor select, debug select, lab mode, trial index, command counter |
| 0x101 | Dyno_Command_2 | Speedgoat -> C2000 | Motor 1 speed reference and optional speed-related limits |
| 0x102 | Dyno_Command_3 | Speedgoat -> C2000 | Motor 2 Iq/load reference and optional load/current limits |

Dyno_Command_1 payload proposal, CAN ID 0x100

| Byte(s) | Signal | Type | Scale / meaning |
| --- | --- | --- | --- |
| 0 | EnablePWM | uint8 | 0 = off, 1 = on |
| 1 | MotorSelect | uint8 | Same meaning as existing Motor1/2 control |
| 2 | DebugSelect | uint8 | 1-7 for Lab 2/Lab 3 debug trial selections |
| 3 | LabMode | uint8 | 0 = manual, 2 = Lab 2 behavior, 3 = Lab 3 CAN mode |
| 4 | TrialIndex | uint8 | 1-7 for prescribed test trial |
| 5 | CommandCounter | uint8 | Increments each command update for dropped-frame detection |
| 6-7 | Reserved | uint8 | Future use |

Dyno_Command_2 payload proposal, CAN ID 0x101

| Byte(s) | Signal | Type | Scale / meaning |
| --- | --- | --- | --- |
| 0-1 | SpeedRefRPM_i16 | int16 | 1 RPM/count or 0.1 RPM/count; choose one and document it |
| 2-3 | SpeedRefPU_i16 | int16 | Optional per-unit speed reference |
| 4-5 | SpeedRampLimit_i16 | int16 | Optional ramp limiter or slew-rate setting |
| 6-7 | Reserved | uint8 | Future use |

Dyno_Command_3 payload proposal, CAN ID 0x102

| Byte(s) | Signal | Type | Scale / meaning |
| --- | --- | --- | --- |
| 0-1 | Mtr2IqRef_i16 | int16 | Recommended scale: 0.001 A/count or 0.01 A/count |
| 2-3 | IqLimit_i16 | int16 | Optional current limit |
| 4-5 | LoadMode_i16 | int16 | Optional dyno mode or load-scheduler mode |
| 6-7 | Reserved | uint8 | Future use |

## 7.2 C2000 to Speedgoat Telemetry Frames

| CAN ID | Frame name | Direction | Purpose |
| --- | --- | --- | --- |
| 0x200 | Dyno_Status | C2000 -> Speedgoat | Mode, enable state, debug select, fault/status flags |
| 0x201 | Dyno_Debug_1 | C2000 -> Speedgoat | Compatibility frame carrying debug_1 and debug_2 |
| 0x202 | Dyno_Speed | C2000 -> Speedgoat | Motor 1 speed reference and feedback |
| 0x203 | Dyno_Mtr1 | C2000 -> Speedgoat | Motor 1 Pm and Te |
| 0x204 | Dyno_Mtr2 | C2000 -> Speedgoat | Motor 2 Pm and Te |
| 0x205 | Dyno_Iq | C2000 -> Speedgoat | Motor 2 Iq reference and feedback |
| Telemetry frame | Suggested signals | Why it matters for Lab 3 |
| 0x202 Dyno_Speed | SpeedRefRPM, SpeedFbRPM | Validates EPA command tracking and closed-loop behavior |
| 0x205 Dyno_Iq | Mtr2IqRef, Mtr2IqFb | Identifies whether Motor 2 is loading or driving |
| 0x203 Dyno_Mtr1 | Mtr1Pm, Mtr1Te | Shows Motor 1 drive output when Motor 2 Iq is positive |
| 0x204 Dyno_Mtr2 | Mtr2Pm, Mtr2Te | Shows Motor 2 drive/assist output when Motor 2 Iq is negative |
| 0x200 Dyno_Status | Enable, lab mode, faults, counters | Confirms safe command state and detects communication failure |

# 8. Corrected Motor 2 Iq Sign Convention

Lab 3 must preserve the corrected sign convention identified during Lab 2 interpretation. This convention should be encoded in Speedgoat logging and analysis logic, not merely stated in the report.

| Motor 2 Iq sign | Motor 2 role | Main useful mechanical power signal |
| --- | --- | --- |
| Positive Motor 2 Iq | Motor 2 acts as the load / dyno | Motor 1 Pm is meaningful |
| Negative Motor 2 Iq | Motor 2 acts as the driver / assist motor | Motor 2 Pm is meaningful |
| Near zero Motor 2 Iq | Transition or little useful load/drive action | Use caution; readings may be small/noisy |

validMotor1Drive = mtr2Iq > 0;   % Motor 2 loads shaft; Motor 1 Pm meaningful
validMotor2Drive = mtr2Iq < 0;   % Motor 2 drives/assists; Motor 2 Pm meaningful

Pm1_valid = Pm1;
Pm2_valid = Pm2;

Pm1_valid(~validMotor1Drive) = NaN;
Pm2_valid(~validMotor2Drive) = NaN;

# 9. Speedgoat Model Structure

The Speedgoat model should generate the EPA drive cycle, compute the desired Motor 1 speed reference, compute or replay the Motor 2 Iq/load schedule, transmit command frames, receive telemetry frames, and log all Lab 3 data.

EPA drive-cycle source
        |
        v
Vehicle-speed-to-motor-RPM conversion
        |
        v
Motor 1 speed reference -----------+
                                    |
Motor 2 Iq/load scheduler ----------+--> CAN command packer --> Speedgoat CAN TX
                                    |
Enable, LabMode, TrialIndex --------+

Speedgoat CAN RX
        |
        v
CAN unpack / DBC decode
        |
        +--> speed tracking plot
        +--> Motor 2 Iq plot
        +--> Motor 1 Pm and Te plot
        +--> Motor 2 Pm and Te plot
        +--> corrected region classification
        +--> data logging for final Lab 3 analysis

# 10. C2000 Target Model Changes

## 10.1 Add CAN Receive Path Without Removing SCI

The safest first change is to add a CAN receive path in parallel with the existing SCI command path. A manual switch or variant control can select whether the motor-control commands come from SCI or CAN. This preserves a fallback path while CAN is being debugged.

Existing SCI command path -----+
                            |
                            +--> Source select --> existing motor-control command signals
                            |
New CAN command path --------+

- Add C28x CAN Receive blocks for CAN IDs 0x100, 0x101, and 0x102.

- Decode the received payload into EnablePWM_CAN, SpeedRefRPM_CAN, Mtr2IqRef_CAN, DebugSelect_CAN, and LabMode_CAN.

- Route decoded signals through scaling/saturation blocks before they enter the existing FOC/dyno command logic.

- Keep PWM disabled during the first decode tests.

## 10.2 Add CAN Transmit Telemetry

After command decoding works, add C28x CAN Transmit blocks for telemetry. Begin with one simple frame, then add frames until the full Lab 3 signal set is available on Speedgoat.

1.  Transmit 0x200 Dyno_Status with command counter and enable state.

2.  Transmit 0x202 Dyno_Speed with speed reference and speed feedback.

3.  Transmit 0x205 Dyno_Iq with Motor 2 Iq reference and feedback.

4.  Transmit 0x203 and 0x204 with Motor 1/Motor 2 Pm and Te.

5.  Optionally retain 0x201 Dyno_Debug_1 as a compatibility frame for the old debug_1/debug_2 workflow.

## 10.3 Remove SCI Dependency Only After CAN Validation

SCI should not be removed until CAN command reception, telemetry transmission, safety timeout, and at least one low-speed motor test have passed. The target model can keep SCI as a backup, but Lab 3 deliverables should use CAN as the primary interface.

# 11. Safety and Fault Handling

| Safety feature | Recommended implementation | Purpose |
| --- | --- | --- |
| Command timeout | If no valid command counter update arrives within a fixed timeout, disable PWM or command zero speed/Iq | Prevents runaway if CAN communication stops |
| Enable gating | Require EnablePWM = 1 and valid LabMode before motor commands are accepted | Avoids accidental motor startup |
| Speed saturation | Clamp SpeedRefRPM to safe lab limits | Prevents over-speed |
| Iq saturation | Clamp Mtr2IqRef to safe lab current limits | Prevents excessive torque/current |
| Fault telemetry | Send fault/status bits in 0x200 Dyno_Status | Lets Speedgoat stop the test and annotate logs |
| Bring-up mode | Run CAN decode with PWM disabled first | Validates communication without motion |

# 12. Recommended Lab 3 Implementation Sequence

| Phase | Name | Test action | Pass condition |
| --- | --- | --- | --- |
| 1 | CAN physical/link bring-up | Connect Speedgoat and F28069M CAN bus, set baud rate, send simple command frame | No bus errors; frames visible on receive side |
| 2 | CAN echo test | Speedgoat sends command counter; C2000 echoes it in 0x200 | Returned counter matches transmitted counter |
| 3 | Command decode test | Send EnablePWM = 0, SpeedRef = 0, IqRef = 0 | C2000 decodes fields correctly while PWM remains off |
| 4 | Low-speed command test | Send EnablePWM = 1 and a small speed reference | Motor 1 tracks safely at low speed |
| 5 | Load sign test | Apply small positive Motor 2 Iq | Motor 2 acts as load and Motor 1 Pm becomes meaningful |
| 6 | Assist sign test | Apply small negative Motor 2 Iq | Motor 2 acts as driver/assist and Motor 2 Pm becomes meaningful |
| 7 | EPA cycle test | Run full Lab 3 EPA drive-cycle command over CAN | All required telemetry is logged on Speedgoat |

# 13. Exact-Copy Mode Versus Final-Speedgoat Mode

| Option | Description | Advantages | Limitations |
| --- | --- | --- | --- |
| Option A: exact-copy debug selector | Speedgoat sends DebugSelect = 1-7; C2000 returns debug_1/debug_2 like the SCI workflow | Closest to current model; easiest to compare against Lab 2 | Requires repeated trials; less efficient; data alignment is harder |
| Option B: continuous telemetry stream | C2000 continuously transmits speed, Iq, Pm, and Te frames | One run can capture all required Lab 3 data; better aligned; more professional architecture | Requires more CAN frames and more model integration work |
| Recommended path Build Option A first for confidence and equivalence with the existing SCI model. Then upgrade to Option B for the final Lab 3 Speedgoat implementation. |

# 14. Timing and Data-Rate Planning

CAN update rates should be chosen to match the purpose of each signal. Commands do not need to update at the PWM rate. Telemetry should be fast enough to capture drive-cycle behavior, but not so fast that it overloads the bus or the target model.

| Signal group | Suggested rate | Notes |
| --- | --- | --- |
| Command frames 0x100-0x102 | 50-100 Hz | 10-20 ms is usually enough for EPA drive-cycle supervisory commands |
| Status frame 0x200 | 10-50 Hz | Faults, mode, and counters do not need high-rate transmission |
| Speed telemetry 0x202 | 50-100 Hz | Needed for tracking analysis |
| Iq telemetry 0x205 | 50-100 Hz | Needed for load/assist classification |
| Pm/Te telemetry 0x203-0x204 | 50-100 Hz | Needed for power/torque plots and energy estimates |

# 15. Scaling and Byte Packing Guidance

Use integer-scaled engineering signals instead of raw floating-point values for the first CAN implementation. This keeps frames compact and makes DBC documentation simple.

| Physical signal | Recommended encoded type | Example scale | Example range |
| --- | --- | --- | --- |
| Speed reference / feedback | int16 | 1 RPM/count | -32768 to 32767 RPM |
| Iq reference / feedback | int16 | 0.001 A/count or 0.01 A/count | +/-32.767 A or +/-327.67 A depending on scale |
| Torque Te | int16 | 0.001 N*m/count | +/-32.767 N*m |
| Mechanical power Pm | int16 or int32 | 0.1 W/count or 1 W/count | Choose based on expected peak power |
| Counters/modes | uint8 | 1 count/bit-coded mode | 0-255 |

Important scaling rule: Document every scale factor in one table and use the same scale in Speedgoat, C2000 decode logic, logging scripts, and student instructions.

# 16. Lab 3 Student Procedure

1.  Verify the CAN wiring, termination, transceiver path, and baud rate.

2.  Open the Speedgoat model and confirm CAN command frames are configured with the selected IDs.

3.  Open the C2000 target model and confirm C28x CAN Receive/Transmit blocks are configured for CAN_A.

4.  Run the CAN echo test with PWM disabled.

5.  Run the command decode test and confirm the C2000 decodes speed reference, Iq reference, enable, and debug selection correctly.

6.  Enable the low-speed command test and verify safe Motor 1 speed tracking.

7.  Apply small positive Motor 2 Iq and confirm the corrected load convention.

8.  Apply small negative Motor 2 Iq and confirm Motor 2 driver/assist behavior.

9.  Run the EPA-style drive cycle from Speedgoat over CAN.

10.  Log speed, Iq, Pm, Te, status, and command counters in Speedgoat.

11.  Use the corrected sign convention to mask and interpret Motor 1 and Motor 2 power.

12.  Submit plots, communication validation results, and a short architecture discussion.

# 17. Required Lab 3 Plots

| Plot | Signals | Purpose |
| --- | --- | --- |
| 1. CAN command verification | Transmitted vs echoed command counter | Proves reliable command transfer |
| 2. Speed tracking | Motor 1 speed reference vs Motor 1 speed feedback | Verifies EPA command tracking through CAN |
| 3. Motor 2 Iq | Motor 2 Iq reference vs feedback | Identifies load/driver regions |
| 4. Motor 1 power/torque | Mtr1 Pm and Te | Shows Motor 1 output during positive Motor 2 Iq loading |
| 5. Motor 2 power/torque | Mtr2 Pm and Te | Shows Motor 2 output during negative Motor 2 Iq driving/assist |
| 6. Both motor power | Mtr1 Pm and Mtr2 Pm | Compares shaft power direction and transfer behavior |
| 7. Both motor torque | Mtr1 Te and Mtr2 Te | Compares torque direction and loading behavior |
| 8. CAN timing/status | Receive timestamps, missed counters, fault/status bits | Validates the communication layer |

# 18. Required Deliverables

- Block diagram showing SCI-to-CAN conversion architecture.

- CAN message table with IDs, directions, signal names, byte positions, data types, and scale factors.

- Screenshot or log proving CAN echo test success.

- Speedgoat log of the EPA drive-cycle run.

- Plots listed in Section 17.

- Short explanation of why C28x CAN blocks are used on the F28069M target rather than generic host CAN blocks.

- Short explanation of the corrected Motor 2 Iq sign convention.

- Discussion of any dropped frames, timeout behavior, bus errors, or scaling problems.

- Final recommendation for whether the model should keep debug-selector mode or move to continuous telemetry streaming.

# 19. Troubleshooting Guide

| Problem | Likely cause | Recommended check |
| --- | --- | --- |
| No CAN messages received | Baud-rate mismatch, wiring issue, no termination, wrong CAN peripheral | Verify CANH/CANL, 120 ohm termination, CAN_A settings, and Speedgoat baud rate |
| Frames received but decoded values are wrong | Byte order, signed/unsigned type, or scale mismatch | Check endian convention, int16 conversion, and scale table |
| Motor starts unexpectedly | Enable signal not gated or stale command accepted | Require valid command counter and EnablePWM before using references |
| Motor does not move during command test | CAN decoded but not routed to FOC command path | Check source-select switch from SCI/CAN to existing command signals |
| Telemetry values freeze | C2000 transmit not triggered, CAN mailbox stuck, or Speedgoat filter mismatch | Check transmit sample time, CAN ID filters, and status frame counter |
| Bus errors occur during motor operation | Noise, grounding, termination, long cable, or high baud rate | Shorten bus, lower baud rate, verify cable and ground strategy |
| Power sign interpretation looks backward | Motor 2 Iq convention or Pm sign convention mismatch | Repeat low positive/negative Iq sign test and update masks accordingly |

# 20. Final Lab 3 Success Criteria

- Speedgoat can send speed and Iq commands to the F28069M target over CAN.

- The F28069M target can decode CAN commands and route them into the existing dyno control signals.

- The F28069M target can transmit speed, Iq, Pm, Te, and status telemetry over CAN.

- Speedgoat can log all required Lab 3 telemetry with timestamps and command counters.

- The same EPA-style drive-cycle behavior from Lab 2 can be reproduced without relying on SCI host control.

- Positive Motor 2 Iq is correctly interpreted as Motor 2 load and Motor 1 useful-Pm region.

- Negative Motor 2 Iq is correctly interpreted as Motor 2 driver/assist and Motor 2 useful-Pm region.

- The system has a safe timeout or fallback behavior if CAN commands stop.

# Appendix A. Minimal Bring-Up Message Set

| Purpose | Transmit frame | Receive/expected frame | Pass condition |
| --- | --- | --- | --- |
| Echo test | 0x100 with CommandCounter = N | 0x200 with CommandCounterEcho = N | Counter matches |
| Command decode | 0x101 SpeedRefRPM = 0; 0x102 Iq = 0 | 0x200 status valid, no PWM enable | Decoded values match and motor remains off |
| Low-speed run | 0x101 SpeedRefRPM = 200; EnablePWM = 1 | 0x202 speed feedback increases smoothly | Motor responds safely |
| Positive-Iq load check | 0x102 Mtr2IqRef > 0 | 0x205 Iq positive, 0x203 Mtr1 Pm meaningful | Sign convention confirmed |
| Negative-Iq driver check | 0x102 Mtr2IqRef < 0 | 0x205 Iq negative, 0x204 Mtr2 Pm meaningful | Sign convention confirmed |

# Appendix B. Suggested Simulink Subsystems

| Subsystem | Model side | Contents |
| --- | --- | --- |
| CAN Command Generator | Speedgoat | EPA speed profile, Iq/load schedule, enable logic, lab mode, counters |
| CAN Command Pack | Speedgoat | CAN Pack blocks or manual byte packing for 0x100-0x102 |
| CAN Telemetry Receive | Speedgoat | CAN Receive and CAN Unpack blocks for 0x200-0x205 |
| CAN Command Decode | C2000 target | C28x CAN Receive blocks, byte unpacking, scaling, saturation, timeout logic |
| CAN Telemetry Encode | C2000 target | Signal selection, scaling, C28x CAN Transmit blocks |
| SCI/CAN Source Select | C2000 target | Manual switch or variant subsystem to select legacy SCI or new CAN inputs |

# Appendix C. Source Notes

The Lab 3 concept is based on the existing Lab 2 dual-motor dyno procedure and the corrected Motor 2 Iq sign convention. The CAN workflow follows the general Simulink CAN pattern of configuring CAN, packing messages, transmitting, receiving, and unpacking messages. On the C2000 target, the corresponding embedded implementation should use the C28x CAN block family for the F28069M hardware target.

# Appendix D. Reference Screenshots From Current Model

Figure D-1. Current host-side SCI transmit/receive pathway that Lab 3 replaces with CAN command and telemetry paths.

Figure D-2. Current C2000 target SCI transmit block area, indicating where CAN telemetry transmission should be added for Lab 3.