PMSM Dual-Motor Dyno (F28069M)

Overview

- Dual PMSM dynamometer using TI C2000: two power & one microcontroller board

- Motor 1 = drive, Motor 2 = load

- Simulink host + embedded FOC control

Project Structure

TI-Piccolo Motor Drive Testbed/
├── Documentation/
│   ├── BOOSTXL-DRV8305EVM/
│   ├── LAUNCHXL-F28069M/
│   ├── Development notes/
│   ├── BoM.xlsx
│   └── PINOUT.xlsx
│
├── Lab/
│   ├── Lab 1 - Motor 1 efficiency/
│   ├── Lab 2 - EPA drive cycle/
│   └── Lab 3 - Speedgoat CAN bus/
│
├── Setup/
│   ├── Physical Setup.docx
│   ├── Software Setup.docx
│   └── Software.zip
│
├── Current Progress
└── Read Me.docx

Hardware

- LAUNCHXL-F28069M: C2000-based launchpad

- 2× BOOSTXL-DRV8305EVM: Three-phase inverter boards

- 2× PMSM motors + encoders

- 24 V supply (≥5 A)

First-Time Setup

- Follow the provided startup/setup documents FIRST (Physical Setup.docx, Software Setup.docx)

- Assemble hardware (LaunchPad + BoostPacks + motors)

- Verify 24 V polarity and wiring

- Connect encoders and USB

- Install MATLAB

- Run c2000setup & install CCS, C2000Ware, and ___

Control

- FOC (Clarke + Park)

- PI current loops (Id/Iq)

- 2-phase current sensing + 3rd reconstruction

Run Procedure

- Open host model

- Build & deploy to F28069M

- Run and command speed/load

Data

- Log RPM, Iq, voltages, torque

- Use save_debug_session

- Perform RPM and load sweeps

Debug & Validation

- Check 3-phase & encoder connections

- Start from a working model and redit

- Ensure Launchpad appears in device manager

- Validate encoder feedback (Position & RPM on the debug signals)

- Verify paced timing is enabled and provide enough power on the DC bus to prevent glitches

Power

- P_mech = Te × ω

- P_elec = 3/2 × Vq × Iq

Notes

- Verify polarity before power

- ADC synchronized to PWM

- SCI used for streaming