# PRIMARY SETUP (PHYSICAL CONNECTIONS)

## Presetup

Stack BoostPacks onto the LaunchPad (F28069M). There is only one way to stack them.

Download MATLAB R2023b + Simulink.

## 1. PSU

- Plug the 24V 5A PSU into the wall.

- Plug PIGTAILS into the PSU's DC barrel jack.

- Screw PIGTAILS  into PVDD and GND.

- Ensure VDD+ is 24VDC and GND is GND; otherwise, the board will blow upon powering. Double-check with DMM as needed.

- If it helps, GND is always closest to the edge of the board.

- Use a jumper to connect VDD+ and GND from power board #1 to power board #2.

3-Phase Terminals

A-B-C

Input Power

VM+  COM

Phase Half-Bridge Power FETs

Filtering Capacitor

- See red and white jumpers in photo.

## 2. Encoder Connections

- J4 – the encoder female pins connect:

- For Motor #1 → J1–J4.

- For Motor #2 → J5–J8.

- Do NOT connect J10. Use J4 for both motors for the DYNO setup.

## 3. Motor Wiring

- Phase leads → blue terminals.

- Red → MOTA.

- White → MOTB.

- Black → MOTC.

- Green → GND → DNC.

- Capstone docs use different wiring; ignore and use this wiring.

- Ensure correct Motor # per board (Motor #1 connects to QEP_A and then MOTA, B, and C on power board #1. Motor #2 connects to QEP_B and then MOTA, B, and C on power board #2).

- See three phases in photo above for reference.

- Repeat for each motor on each BOOSTXL.

## 4. USB Link

Connect USB link to host computer.

## 5. Final Check

- Check polarity to mitigate failure.

## Launch Instructions

See software setup instructions.