# PMSM FOC DYNO HOST MODEL – DEBUG SIGNAL NOTES

The Debug signals menu selects which internal variables from the motor control system are sent to the host model for visualization. These signals come from the field-oriented control (FOC) algorithm running on the C2000 controller.

Each option corresponds to a different group of physical or control variables.

The system contains two motors:

Motor 1
 • The primary motor under speed control.

Motor 2
 • The dynamometer (load motor) used to apply torque to Motor 1.

# Mtr1: Speed ref & Speed feedback

These signals describe the speed control loop for Motor 1.

Speed_ref
 The commanded motor speed provided to the speed controller.

Units
 Typically RPM (revolutions per minute).

Meaning
 This is the desired speed the controller attempts to maintain.

Speed_feedback
 The measured mechanical speed of the motor.

Units
 RPM.

Meaning
 This value comes from the rotor position sensor or estimator and represents the actual motor speed.

Purpose of observing this signal
 Used to evaluate speed regulation performance and how well the motor tracks the commanded speed.

# Mtr1: Id ref & Id feedback

These signals describe the d-axis current control loop.

Id_ref
 Reference d-axis current.

Units
 Amps.

Meaning
 The commanded magnetic flux current component.

In most PMSM applications
 Id_ref ≈ 0 A

Reason
 Because surface PMSMs produce optimal torque when d-axis current is near zero.

Id_feedback
 Measured d-axis current.

Units
 Amps.

Meaning
 The actual current component aligned with the rotor magnetic field.

Purpose
 Used to verify that the d-axis current regulator maintains the commanded value.

# Mtr1: Iq ref & Iq feedback

These signals describe the torque producing current loop.

Iq_ref
 Reference q-axis current.

Units
 Amps.

Meaning
 The commanded torque current generated by the speed controller.

Iq_feedback
 Measured q-axis current.

Units
 Amps.

Meaning
 The actual torque-producing current measured in the motor.

Physical meaning

Torque is approximately proportional to Iq:

T ≈ Kt × Iq

Where

T = motor torque (Nm)
 Kt = torque constant (Nm/A)

Purpose
 Used to evaluate torque control performance.

# Mtr1: Vd & Vq

These signals represent the dq voltage components applied by the inverter.

Vd
 d-axis voltage.

Units
 Volts.

Meaning
 Voltage component aligned with the rotor magnetic axis.

Purpose
 Used to control magnetic flux and compensate cross-coupling.

Vq
 q-axis voltage.

Units
 Volts.

Meaning
 Voltage component perpendicular to the rotor flux.

Purpose
 Produces torque and compensates back-EMF.

Electrical power can be calculated using dq voltages and currents:

P = \frac{3}{2}(V_d I_d + V_q I_q)

# Mtr1: Ia & Ib feedback

These signals represent the phase currents measured directly from the inverter.

Ia
 Phase A current.

Units
 Amps.

Ib
 Phase B current.

Units
 Amps.

Ic can be calculated as

Ic = −Ia − Ib

These are the raw current measurements before transformation into the dq reference frame.

Purpose
 Used to observe inverter current waveforms and verify current sensing.

# Mtr1: Pm & Te

These signals describe mechanical and electromagnetic power quantities.

Pm
 Mechanical power.

Units
 Watts.

Meaning
 Mechanical shaft power produced by the motor.

Calculated as

Pm = T × ω

Where

T = torque (Nm)
 ω = mechanical angular velocity (rad/s)

Te
 Electromagnetic torque.

Units
 Newton-meters (Nm).

Meaning
 Torque produced by interaction of stator currents and rotor magnetic field.

# Mtr2: Id ref & Id feedback

Same meaning as Motor 1 but for the load motor.

Motor 2 is used as a dynamometer.

It produces controlled torque that loads Motor 1.

# Mtr2: Iq ref & Iq feedback

Same as Motor 1 but applied to the dynamometer motor.

Iq_ref
 Torque command applied to the load motor.

Iq_feedback
 Actual torque-producing current in the load motor.

# Mtr2: Vd & Vq

Voltage components applied to the load motor.

Used by the FOC algorithm to generate the required torque load.

# Mtr2: Ia & Ib feedback

Measured phase currents of the dynamometer motor.

# Mtr2: Pm & Te

Mechanical power and electromagnetic torque produced by the load motor.

Used to analyze energy transfer between the motors.

# Mtr1 & Mtr2: Pm

Displays mechanical power of both motors simultaneously.

Used to observe energy flow in the dynamometer system.

Typical observation

Motor 1 power ≈ −Motor 2 power + system losses

Meaning

One motor drives the other.

# Mtr1 & Mtr2: Te

Displays electromagnetic torque of both motors.

Useful for analyzing torque interaction between motors.

# Mtr1 & Mtr2: Pos

Rotor mechanical position of both motors.

Units
 Electrical radians or mechanical radians depending on implementation.

Purpose
 Used for rotor alignment and Park transformation.

# Mtr1 & Mtr2: Ia

Displays phase current measurements for both motors simultaneously.

Useful for comparing current waveforms.

# Mtr1 Speed ref & Mtr2 Speed fb

Displays

Motor 1 speed reference

and

Motor 2 measured speed.

Used for verifying dynamometer coupling behavior.

# Summary of Key Variables

Speed_ref
 Commanded motor speed.

Speed_feedback
 Measured motor speed.

Id_ref
 Reference d-axis current.

Id_feedback
 Measured d-axis current.

Iq_ref
 Reference q-axis current (torque command).

Iq_feedback
 Measured q-axis current.

Vd
 d-axis voltage.

Vq
 q-axis voltage.

Ia
 Phase A current.

Ib
 Phase B current.

Pm
 Mechanical output power.

Te
 Electromagnetic torque.

Pos
 Rotor mechanical position.