# PMSM FIELD-ORIENTED CONTROL (FOC) PIPELINE NOTES

These notes describe the full signal flow used to control a Permanent Magnet Synchronous Motor (PMSM) using Field-Oriented Control (FOC). The control system converts measured phase currents into a rotating reference frame aligned with the rotor so torque and flux can be controlled independently.

The pipeline consists of several stages:

- Current measurement

- Clarke transformation

- Park transformation

- Current control loops

- Inverse Park transformation

- Space Vector PWM generation

- Inverter switching

# 1. Phase Current Measurement

The inverter measures two phase currents from the motor.

Ia
 Phase A current.

Units
 Amps.

Ib
 Phase B current.

Units
 Amps.

Ic
 Phase C current.

Units
 Amps.

Because the three phase currents must sum to zero:

Ic = −Ia − Ib

Only two currents need to be measured.

These values come from current sensors placed in the inverter phases.

# 2. Clarke Transformation (abc → αβ)

The Clarke transform converts the three phase current system into a two-axis stationary reference frame.

The outputs are:

Iα (Ialpha)
 Current component along the α axis.

Units
 Amps.

Iβ (Ibeta)
 Current component along the β axis.

Units
 Amps.

This transformation converts the 3-phase system into a 2-axis orthogonal system in a stationary frame.

Purpose
 Simplifies analysis of the motor currents.

# 3. Park Transformation (αβ → dq)

The Park transform rotates the stationary αβ frame so it aligns with the rotor magnetic field.

The rotor electrical angle is required:

θe (theta_e)
 Electrical rotor angle.

Units
 Radians.

Using this angle, the αβ currents are rotated into the dq reference frame.

Outputs

Id
 d-axis current.

Units
 Amps.

Meaning
 Current aligned with the rotor magnetic field.

Controls magnetic flux.

Iq
 q-axis current.

Units
 Amps.

Meaning
 Current perpendicular to rotor flux.

Produces torque.

Key concept

Torque is primarily proportional to Iq:

T ≈ Kt × Iq

Where

T = torque (Nm)
 Kt = motor torque constant (Nm/A)

# 4. Current Control Loops

Two independent PI controllers regulate the currents.

D-axis current controller

Inputs

Id_ref
 Reference d-axis current.

Units
 Amps.

Id_feedback
 Measured d-axis current.

Units
 Amps.

Output

Vd
 d-axis voltage command.

Units
 Volts.

Purpose

Controls magnetic flux.

In most PMSM systems

Id_ref ≈ 0

because surface PMSMs produce maximum torque per amp when Id is zero.

Q-axis current controller

Inputs

Iq_ref
 Reference q-axis current.

Units
 Amps.

Iq_feedback
 Measured q-axis current.

Units
 Amps.

Output

Vq
 q-axis voltage command.

Units
 Volts.

Purpose

Controls torque output of the motor.

# 5. Speed Control Loop

The speed loop generates the torque command for the motor.

Inputs

Speed_ref
 Desired motor speed.

Units
 RPM.

Speed_feedback
 Measured motor speed.

Units
 RPM.

Controller

Speed PI controller.

Output

Iq_ref
 Torque current reference.

Units
 Amps.

This means

Speed controller → produces torque command → which becomes Iq_ref.

# 6. Voltage Vector Calculation

The outputs of the current controllers are:

Vd
 d-axis voltage.

Vq
 q-axis voltage.

These represent the desired voltage vector in the rotor reference frame.

Electrical power can be calculated as:

P = \frac{3}{2}(V_d I_d + V_q I_q)

When Id ≈ 0, this simplifies to

P ≈ (3/2) Vq Iq

# 7. Inverse Park Transformation (dq → αβ)

The dq voltage vector must be converted back into the stationary frame.

Inputs

Vd
 Vq
 θe (electrical rotor angle)

Outputs

Vα
 Voltage along α axis.

Vβ
 Voltage along β axis.

These represent the desired stator voltage vector.

# 8. Space Vector PWM (SVPWM)

The αβ voltage vector is converted into duty cycles for the three inverter phases.

Outputs

DutyA
 PWM duty cycle for phase A.

DutyB
 PWM duty cycle for phase B.

DutyC
 PWM duty cycle for phase C.

Units
 Percent or normalized value (0-1).

Purpose

Generates the switching signals required to produce the requested voltage vector.

# 9. Inverter Switching

The inverter consists of six MOSFETs arranged as three half-bridges.

Each phase is driven by complementary PWM signals.

The inverter converts DC bus voltage into three-phase AC voltage for the motor.

The resulting phase voltages drive the motor currents.

# 10. Motor Electromagnetic Output

The interaction between stator currents and the rotor magnetic field produces torque.

Key outputs

Te
 Electromagnetic torque.

Units
 Newton-meters.

Pm
 Mechanical power.

Units
 Watts.

Mechanical power is calculated as

Pm = T × ω

Where

T = torque (Nm)
 ω = mechanical angular velocity (rad/s)

# 11. Dyno System Interaction

In the dynamometer setup there are two motors.

Motor 1
 Speed controlled motor.

Motor 2
 Torque controlled load motor.

Energy transfer occurs between them.

Typical relationship

Motor 1 mechanical power ≈ − Motor 2 mechanical power + losses.

One motor drives the other.

# Full Signal Flow Summary

Sensor currents
 ↓
 Ia, Ib measurement
 ↓
 Clarke transform
 ↓
 Iα, Iβ
 ↓
 Park transform
 ↓
 Id, Iq
 ↓
 Current PI controllers
 ↓
 Vd, Vq
 ↓
 Inverse Park transform
 ↓
 Vα, Vβ
 ↓
 Space Vector PWM
 ↓
 PWM duty cycles
 ↓
 Inverter switching
 ↓
 Motor torque and speed